pre-construction plans and documentation: Detailed construction and monitoring programme:

benthic monitoring plan

| Date | 08 June 2021 |
| --- | --- |
| Document reference | 003552655 |
| Revision | 06 |

| Project |  |  | Sofia Offshore Wind Farm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sub-Project or Package |  |  | Consents |  |  |  |
| Document Title |  |  | Detailed Construction and Monitoring Programme Benthic Monitoring Plan |  |  |  |
| Document Reference |  |  | 003552655 |  |  |  |
| Revision |  |  | 06 |  |  |  |
| Supplier Reference No |  |  | - |  |  |  |
| This document and any information therein are confidential property of Sofia Offshore Wind Farm Limited and without infringement neither the whole nor any extract may be disclosed, loaned, copied or used for manufacturing, provision of services or other purposes whatsoever without prior written consent of Sofia Offshore Wind Farm Limited, and no liability is accepted for loss or damage from any cause whatsoever from the use of the document. Sofia Offshore Wind Farm Limited retains the right to alter the document at any time unless a written statement to the contrary has been appended. |  |  |  |  |  |  |
| Revision | Date | Status/Reason for Issue |  | Originator | Checked | Approved |
| 01 | 09/09/2020 | For submission to MMO for approval |  | Ross Griffin (Ocean Ecology Limited) | Harriet Thomas and Julie Drew-Murphy (SOWFL) | Kim Gauld-Clark (SOWFL) |
| 02 | 16/11/2020 | For resubmission to MMO for approval |  | Ross Griffin (Ocean Ecology Limited) | Harriet Thomas and Julie Drew-Murphy (SOWFL) | Kim Gauld-Clark (SOWFL) |
| 03 | 15/01/2021 | For resubmission to MMO for approval following further comments and inclusion of nearshore sampling locations |  | Ross Griffin (Ocean Ecology Limited) | Harriet Thomas and Julie Drew-Murphy (SOWFL) | Kim Gauld-Clark (SOWFL) |
| 04 | 01/03/2021 | For resubmission to MMO for approval following further comments |  | Ross Griffin (Ocean Ecology Limited) | Harriet Thomas and Julie Drew-Murphy (SOWFL) | Kim Gauld-Clark (SOWFL) |
| 05 | 15/06/2021 | For resubmission to MMO for approval following addition of offshore and SRI sampling arrays |  | Ross Griffin (Ocean Ecology Limited) | Julie Drew-Murphy (SOWFL) | Kim Gauld-Clark (SOWFL) |
| 06 | 26/07/2021 | For resubmission to MMO for approval following further MMO/NE comments |  | Ross Griffin (Ocean Ecology Limited) | Julie Drew-Murphy (SOWFL) | Clare Davies (SOWFL) |

Contents

1.	Overview	6

1.1	Introduction	6

1.2	Purpose and Scope of Document	6

1.3	Monitoring Requirements	8

1.4	in Principle Monitoring Plan and guiding principles	12

1.5	Existing and prospect data	12

1.6	Consultation process	12

2.	project description	15

2.1	overview	15

2.2	Ownership of generation and transmission assets	16

3.	Development of the benthic Monitoring Plan	17

3.1	Introduction	17

3.2	findings of the environmental statement	18

3.3	Findings of the Report to Inform Appropriate Assessment	18

3.4	Non-material Change Applications and deemed Marine Licence Variations	19

3.5	Benthic monitoring note	19

3.6	BMP: Supporting Studies document	19

3.7	Characterisation sampling mapping	20

3.8	Engagement with senckenberg research institute	20

3.9	relevant guidance documents	21

4.	Aproach To benthic Monitoring	21

4.1	Features of focus	21

4.2	Feature of Focus i: sediments	21

Objective	21

Survey design	21

Approach	24

Cumulative Impacts	25

4.3	Feature of Focus II: Biogenic/geogenic reef	26

Objective	26

Approach	26

4.4	Feature of Focus iII: Invasive non-native species	27

Objective	27

4.5	Monitoring Schedule	28

4.6	Reporting	28

5.	References	30

APPENDIX A.	Benthic monitoring objectives for the Dogger Bank SAC set out by Natural England	32

APPENDIX B.	EXISTING DATA USED IN THE DEVELOPMENT OF THE BMP.	36

APPENDIX C.	Offshore In Principle Monitoring Plan and rationale of monitoring presented within this benthic Monitoring  plan	42

APPENDIX D.	EUNIS mapping	45

APPENDIX E.	Valued Ecological Receptor (VER) categories	46

APPENDIX F.	Valued Ecological Receptor (VER) protection status and conservation interest and their importance/value within the study area.	47

APPENDIX G.	Summary of predicted effects of SOWF on benthic habitats.	49

APPENDIX H.	Associated documents	51

APPENDIX I.	final sampling design – nearshore (out to Kilometre Point 10)	52

APPENDIX J.	final sampling design – Offshore (beyond Kilometre Point 10)	56

APPENDIX K.	Senckenberg Research Institute (SRI) long-term and REWIND sampling design	58

APPENDIX L.	Predictive Habitat modelling report	62

Tables

Table 1-1 dML Conditions (as varied in April 2019) discharged in Full or Part, through the provision of the Benthic Monitoring Plan.	7

Table 1-2 Pre- and Post-Construction Plans and Documents: Relevant Deemed Marine Licence Conditions	8

Table 1-3 Monitoring requirements based on the dML conditions, IPMP. Dogger Bank SAC monitoring objectives and existing benthic ecology knowledge within the SOWF Order Limits.	11

Table 1-4 Consultation process for benthic monitoring proposals.	13

Table 2-1 Asset ownership of the Wind Farm Operator and Offshore Transmission Owner.	17

Table 4-1 Outline of pre-construction benthic baseline reporting structure.	29

Figures

Figure 2-1 The location and project layout of the Sofia Offshore Wind Farm (SOWF).	15

Figure 2-2 Diagram showing the Grid Entry Point between the Wind Farm Operator and Offshore Transmission Owner systems.	17

Figure 4-1 Proposed arrangement of benthic sampling at each selected WTG location. Note that BRUV sampling will only take place at a subset of WTG locations.	23

Figure I-1 Overview of nearshore BMP DDC transects overlain on 2020 SSS data.	52

Figure I-2 – Location of nearshore DDC transects relative to the proposed HDD exit point options overlain on 2020 MBES data.	53

Figure I-3– Location of nearshore DDC transects out to KP 8 overlain on 2020 MBES data.	54

Appendices

Appendix A – Natural England and JNCC benthic monitoring objectives for the Dogger Bank SAC

Appendix B - Existing and prospect data used in the development of the BMP.

Appendix C - Offshore in Principle monitoring plan and rationale of monitoring presented within this benthic monitoring plan

Appendix D - EUNIS Mapping

Appendix E - Valued Ecological Receptor (VER) categories

Appendix F - Valued Ecological Receptor (VER) protection status and conservation interest and their importance/value within the study area.

Appendix G - Summary of predicted effects of SOWF on benthic habitats

Appendix H - Associated documents

Appendix I – Nearshore sampling locations (out to Kilometre Point 10)

Appendix J – Offshore sampling locations (beyond Kilometre Point 10)

Appendix K – Senckenberg Research Institute (SRI) long-term and REWIND sampling design

Appendix L – Predictive modelling report (provided under separate cover)

# Overview

## Introduction

Sofia Offshore Wind Farm ("SOWF") is located approximately 165 km offshore on the shallow central area of the North Sea known as the Dogger Bank with the export cable landfall in an area between Redcar and Marske-by-the-Sea.. SOWF was developed and consented by the Forewind Limited Consortium and previously known as Dogger Bank Teesside B ("Teesside B"). The Dogger Bank Teesside A and B Offshore Wind Farm Order 2015 ("the DCO") was granted on 4th August 2015 and came into force on the 26th August 2015. The Development Consent Order (DCO) was amended in 2019. The Forewind Limited consortium disbanded and since August 2017, Innogy Renewables UK Limited ("innogy") has held 100% ownership of SOWF under a new subsidiary, Sofia Offshore Wind Farm Limited ("SOWFL").  On 30 June 2020, innogy, and its subsidiary SOWFL, transferred to become part of RWE.

## Purpose and Scope of Document

This document forms part of a suite of documents that will be produced under the requirements set out within Condition 16 of Schedule 9 (Deemed Marine Licence (“dML”) 2: Project B (SOWF) Offshore Generation - Work Nos. 1B and 2T) (Variation No. 2, April 2019) and Condition 14 of Schedule 11 dML 4: Project B (SOWF) Offshore Transmission – Works Nos. 2B, 3B and 2T (Variation No. 2, April 2019). These Conditions require Pre-Construction Plans and Documents to be submitted and approved by the Marine Management Organisation (“MMO”) where relevant in consultation with statutory and non-statutory advisors. The dMLs were varied by the MMO in 2020 (Variation No. 3, September 2020) following the conclusion of the Southern North Sea Special Area of Conservation Review of Consents but no changes to the requirements for benthic monitoring were made to the consents.

This document has been drafted to address the requirements of the DCO dML, insofar as is relevant to the preparation of the benthic monitoring component of the Detailed Construction Monitoring Programme for SOWF under Condition 16(b) of dML Schedule 9 (as varied in April 2019) and Condition 14(b) of dML Schedule 11 (as varied in April 2019). The specific requirements for benthic monitoring are set out within Conditions 21 and 23 of dML Schedule 9 (Variation No. 2, April 2019) and Conditions 19 and 21 of dML Schedule 11 (Variation No. 2, April 2019). Section 1.3 details the legislative context for the Benthic Monitoring Plan (BMP).

The purpose of this document is to:

Provide a clear rationale for the approach taken in developing the BMP (Section 2);

Define the objectives for the benthic monitoring to be undertaken (Sections 4);

Set out the approaches to be used to monitor key benthic habitats (Section 4);

Set out the structure and format for reporting of the benthic monitoring surveys (4.6); and

Set out the programme for the benthic monitoring surveys and reporting (Section 4);

The monitoring objectives and proposed surveys have been defined based on a focused approach to benthic monitoring in consideration of the following:

The Dogger Bank Teesside A and B Deadline IX Appendix 4 Offshore In Principle Monitoring Plan (“IPMP”) (Forewind, 2015) relevant guiding principles (see Section 1.4);

The requirements of the dML (Table 1-1 and Table 1-2);

Benthic monitoring objectives for the Dogger Bank SAC set out by Natural England (see Appendix A);

The specific habitats present in the project construction area (see Appendix B); and

The nature of the infrastructure to be installed (Section 2).

The final sampling design for the pre-construction surveys will be provided to the MMO at least four months before commencement of any survey works. Appendix I and J provide the final sampling design for the nearshore (out to KP 10) and offshore (beyond KP10) sampling design respectively. Appendix K also provides the sampling design for the Senckenberg Research Institute (SRI) sampling which will make up part of the SOWF benthic monitoring plan (see Sections 3.8 and 4.

As set out in Table 1-2, the requirement for post-construction surveys are dependent on the outcome of the pre-construction surveys. However, SOWFL have included for adaptive post-construction monitoring within this BMP. The objectives and the design of the pre-construction survey have therefore considered the need for post-construction monitoring and as such, the BMP can discharge in full, the requirements for benthic monitoring.

Given the proximity of SOWF to the planned Dogger Bank Wind Farm (DBWF) A and B, and subsequently C, being developed by SSE and Equinor (Figure 2-1), discussions are ongoing with DBWF to ensure the data collected for all wind farm projects are comparable. This will help to maximise the value of the data collected and allow for any regional scale impacts to be assessed if required.

Table 1-1 dML Conditions (as varied in April 2019) discharged in Full or Part, through the provision of the Benthic Monitoring Plan.

| DCO Schedule 9: Marine Licence 2: Project B (SOWF) Offshore Generation – Work Nos. 1B and 2T: Part 2 Conditions (Variation No. 2, April 2019) |  |
| --- | --- |
| dML Condition | In Full or Part discharge |
| Pre-construction plans and documentation: Detailed Construction and Monitoring Programme 16(b)(iii) | Part |
| Pre-construction monitoring 21(2)(a), | Full |
| Post-construction surveys 23(2)(c) | Full |
| Post-construction surveys 23(2)(e) | Discharged in full through approval of the SOWFL Geophysical (Bathymetrical) Monitoring Plan (Approved by MMO in 2019) |
| DCO SCHEDULE 11: MARINE LICENCE 4: PROJECT B (SOWF) OFFSHORE TRANSMISSION – WORK NOS. 2B, 3b AND 2T: PART 2 CONDITIONS (VARIATION NO. 2, APRIL 2019) |  |
| dML Condition | In Full or Part discharge |
| Pre-construction plans and documentation: Detailed Construction and Monitoring Programme 16(b)(iii) | Part |
| Pre-construction monitoring 19(2)(a), | Full |
| Post-construction surveys 21(2)(b) | Full |

## Monitoring Requirements

The dMLs stipulate the requirements for benthic monitoring for the Project with further information set out in the IPMP and Chapter 12 of the ES (Forewind 2014). Table 1-2 sets out the detail of Conditions 16(b), 21(2) and 23(2) of dML Schedule 9 (Offshore Generation) (Variation No. 2, April 2019) and of Conditions 14(b), 19(2) and 21(2) of dML Schedule 11 (Offshore Transmission) (Variation No. 2, April 2019) relevant to the BMP.

The monitoring requirements fall into two main categories - those relating to benthic habitats of conservation, ecological or economic importance (BHoCEEI) (including Annex I habitats) and other monitoring targeting particular receptors or focusing on potential impacts that were identified in the ES. The first category (BHoCEEI) can be subdivided into sediment habitats (e.g. sandbanks) and reef habitats (e.g. Sabellaria spinulosa or stony reefs) occurring within the order limits whereas the second category comprises requirements for monitoring of drill disposal mounds, invasive non-native species (INNS) and wider community type structure. Table 1-3 sets out each of these Features of Focus (FoF) and the requirements of the surveys described in the following sections.

It should be noted that this BMP should be read in conjunction with the SOWFL Geophysical (Bathymetrical) Monitoring Plan (GMP) (SOWFL, 2019a) (approved by the MMO in December 2019). The data gathered during the geophysical surveys, as set out in the GMP, will be used in conjunction with the benthic survey approaches described in the following sections to discharge the dML conditions relevant to benthic ecology (namely mapping the location and extent of Annex I habitats), drill disposal mounds (if required) and geophysical monitoring.

It should also be noted that since cumulative impacts from SOWF and DBWF were not predicted in the ES, the monitoring set out in this BMP is focused on key FoF rather than aimed at assessing regional scale cumulative impacts although broad scale regional impacts will be monitored as part of the SRI’s long-term monitoring of the wider Dogger Bank (see Section 3.8).

Table 1-2 Pre- and Post-Construction Plans and Documents: Relevant Deemed Marine Licence Conditions

DCO Schedule 9: ML 2: Project B (SOWF) Offshore Generation – Work Nos. 1B and 2T: Part 2 (as varied in 2019)

Pre-Construction Plans and Documents

Detailed construction and monitoring programme

16(b) a detailed construction and monitoring programme, including details of—

(i) the proposed construction commencement date;

(ii) proposed timings for mobilisation of plant, delivery of materials and installation works; and

(iii) proposed pre-construction surveys, a proposed format and content for a baseline report, construction monitoring, post-construction monitoring and related reporting in consultation with the relevant statutory nature conservation body. The preconstruction survey programme and all pre-construction survey methodologies must be submitted to the MMO for written approval at least 4 months before commencement of any survey works detailed within;

Pre-Construction Monitoring

21.— (1) The undertaker must, in discharging Condition 16(b), and the requirement to prepare a detailed construction and monitoring programme, include details for written approval by the MMO of proposed pre-construction surveys, including methodologies and timings, and a proposed format and content for a pre-construction baseline report. The survey proposals must be in accordance with the principles set out in the offshore in principle monitoring plan and must specify each survey’s objectives and explain how it assists in either informing a useful and valid comparison with the post-construction position or enables the validation or otherwise of key predictions in the environmental statement. The baseline report proposals must ensure that the outcome of the agreed surveys together with existing data and reports are drawn together to present a valid statement of the pre-construction position, with any limitations, and must make clear what post-construction comparison is intended and the justification for this being required.

(2) Subject to receipt from the undertaker of specific proposals pursuant to this Condition, where appropriate and necessary it is expected that the pre-construction surveys will comprise—

(a) an appropriate survey to determine the location and reasonable extent of any benthic habitats of conservation, ecological or economic importance (including Annex 1 habitats) in whole or in part inside the areas within the Order limits in which it is proposed to carry out construction works;

(3) The undertaker must carry out and complete the surveys to be undertaken under subparagraph (1) in a timescale agreed by the MMO.

Post-construction monitoring

23.—(1) The undertaker must, in discharging Condition 16(b), submit details for written approval by the MMO of the 5 post-construction surveys proposed in sub-paragraph (2), including methodologies and timings, and a proposed format, content and timings for providing reports on the results at least 4 months before commencement of any survey works detailed within. The survey proposals must be in accordance with the principles set out in the offshore in principle monitoring plan and specify each survey’s objectives and explain how it assists in either informing a useful and valid comparison with the pre-construction position or enables the validation or otherwise of key predictions in the environmental statement.

(2) Subject to receipt of specific proposals, it is expected that the post-construction surveys will comprise—

(c) dependent on the outcome of the surveys undertaken under Condition 21(2)(a), appropriate surveys to determine the effects of construction activity on any benthic habitats of conservation, ecological or economic importance (including Annex 1 habitats) the environmental statement and to identify the presence of any non-native species and wider community type structure; in whole or in part inside the areas within the Order limits to validate predictions made in the environmental statement and to identify the presence of any non-native species and wider community type structure;

(e) appropriate surveys to determine the change in size and form of the drill disposal mounds over the lifetime of the authorised scheme.

(3) The undertaker must carry out the surveys under sub-paragraph (1) and provide the reports in the agreed format in accordance with the timetable agreed in writing by the MMO following consultation with the relevant statutory nature conservation body.

DCO Schedule 11: ML 4: Project B (SOWF) Offshore Transmission – Work Nos. 2B, 3T and 2T: Part 2 Conditions (as varied in 2019)

Pre-Construction Plans and Documents

Detailed construction and monitoring programme

14(b) a detailed construction and monitoring programme, including details of—

(i) the proposed construction commencement date;

(ii) proposed timings for mobilisation of plant, delivery of materials and installation works; and

(iii) proposed pre-construction surveys, a proposed format and content for a baseline report, construction monitoring, post-construction monitoring and related reporting in consultation with the relevant statutory nature conservation body. The preconstruction survey programme and all pre-construction survey methodologies must be submitted to the MMO for written approval at least 4 months before commencement of any survey works detailed within;

Pre-construction monitoring

19.—(1) The undertaker must, in discharging Condition 14(b), and the requirement to prepare a detailed construction and monitoring programme, include details for written approval by the MMO of proposed pre-construction surveys, including methodologies and timings, and a proposed format and content for a pre-construction baseline report. The survey proposals must be in accordance with the principles set out in the offshore in principle monitoring plan and must specify each survey’s objectives and explain how it assists in either informing a useful and valid comparison with the post-construction position or enables the validation or otherwise of key predictions in the environmental statement. The baseline report proposals must ensure that the outcome of the agreed surveys together with existing data and reports are drawn together to present a valid statement of the pre-construction position, with any limitations, and must make clear what post construction comparison is intended and the justification for this being required.

(2) Subject to receipt from the undertaker of specific proposals pursuant to this Condition, where appropriate and necessary it is expected that the pre-construction surveys will comprise—

(a) an appropriate survey to determine the location and reasonable extent of any benthic habitats of conservation, ecological or economic importance including (Annex 1 habitats) in whole or in part inside the areas within the Order limits in which it is proposed to carry out construction works; and

(3) The undertaker must carry out and complete the surveys to be undertaken under subparagraph (1) in a timescale agreed by the MMO.

Post construction monitoring

21.—(1) The undertaker must, in discharging Condition 14(b), submit details for written approval by the MMO of the 3 post-construction surveys proposed in sub-paragraph (2), including methodologies and timings, and a proposed format, content and timings for providing reports on the results at least 4 months before commencement of any survey works detailed within. The survey proposals must be in accordance with the principles set out in the offshore in principle monitoring plan and specify each survey’s objectives and explain how it assists in either informing a useful and valid comparison with the pre-construction position or enables the validation or otherwise of key predictions in the environmental statement.

(2) Subject to receipt of specific proposals, it is expected that the post-construction surveys will comprise—

(b) dependent on the outcome of the surveys undertaken under Condition 19(2)(a), appropriate surveys to determine the effects of construction activity on any benthic habitats of conservation, ecological or economic importance (including Annex 1 habitats) in whole or in part inside the areas within the Order limits to validate predictions made in the environmental statement and to identify the presence of any non-native species and wider community type structure; and

(3) The undertaker must carry out the surveys under sub-paragraph (1) and provide the reports in the agreed format in accordance with the timetable agreed in writing by the MMO following consultation with the relevant statutory nature conservation body.

Table 1-3 Monitoring requirements based on the dML conditions, IPMP. Dogger Bank SAC monitoring objectives and existing benthic ecology knowledge within the SOWF Order Limits.

| Monitoring | Relevant ML Condition / IPMP Section / SAC Monitoring Objective | Requirement |
| --- | --- | --- |
| Benthic Habitats of Conservation, Ecological and Economic Importance (BHoCEEI) Monitoring |  |  |
| Sediments (Feature of Focus I): - Dogger Bank SAC Annex I Sand Bank feature - Section 41 Priority Habitat ‘subtidal sands and gravels’ - Section 41 Priority Habitat ‘mud habitats in deep water | ▪ Conditions 16(b), 21(2) (a) and 23(2)(c) of dML 2, Schedule 9 (Offshore Generation) (Variation No. 2, April 2019) ▪ Conditions 14(b), 19(2)(a) and 21(2)(b) of dML 4, Schedule 11 (Offshore Transmission) (Variation No. 2, April 2019) ▪ IPMP Section 4.5 marine (benthic) ecology ▪ Dogger Bank SAC Monitoring Objective 1 - 4 | Surveys to determine the location and reasonable extent of any BHoCEEI in whole or in part inside the areas within the Order limits in which it is proposed to carry out construction works. The approach to surveying these sediment habitats is set out in Section 4.2. |
| Biogenic / geogenic reef (Feature of Focus II): - Annex I Sabellaria spinulosa reef - Annex I stony/cobble reef - Section 41 Peat and Clay Exposures* |  | Surveys to determine the location and reasonable extent of any BHoCEEI in whole or in part inside the areas within the Order limits in which it is proposed to carry out construction works. The approach to surveying reef habitats is set out in Section 4.3. |
| Other Targeted / Impact Specific Monitoring |  |  |
| Invasive non-native species (INNS) (Feature of Focus III) | ▪ IPMP Section 4.5 marine (benthic) ecology ▪ Dogger Bank SAC Monitoring Objective 3 | Appropriate consideration shall be given to monitoring the presence of any INNS and wider community type/structure, to identify changes that may affect site integrity, a summary of which is set out in Section 4.4. |
| Drill disposal mounds | ▪ Conditions 23(2)(e) of dML 2, Schedule 9 (Offshore Generation) (Variation No. 2, April 2019) ▪ IPMP Section 4.5 marine (benthic) ecology ▪ Dogger Bank SAC Monitoring Objective 1, 3 & 4 | Appropriate surveys to determine change in size and form of the drill disposal mounds (mud or clay arisings) over the lifetime of the authorised scheme, if drilling has been used for the installation of any foundations. Although the scope and timings for this is to be agreed post consent, and is only required if drilling is used to install foundations. Discharge of this Requirement has been approved through the SOWF Geophysical (Bathymetrical) Monitoring Plan (2019) and is therefore not discussed further in this document. |

## in Principle Monitoring Plan and guiding principles

As set out in the dML conditions, any monitoring for the SOWF project must be undertaken in accordance with the Dogger Bank Teesside A and B Deadline IX Appendix 4 Offshore In Principle Monitoring Plan (“IPMP”) (Forewind, 2015). Appendix C presents the text and monitoring proposed for benthic ecology as detailed in Section 4.5 of the IPMP.  Appendix C also provides the rationale for the monitoring proposed within this document against each of the considerations set out in the IPMP with regards to benthic ecology. The IPMP confirms that the pre-construction surveys should focus on Annex I habitats that may be affected by construction, with appropriate consideration being given to monitoring the presence of any INNS and wider community type/structure, to identify changes that may affect the integrity of the Dogger Banks SAC.

The following general guiding principles have also been used to define the scope of the monitoring proposals set out in this BMP:

The proposed surveys will be “…necessary, relevant to planning, relevant to the permitted development, enforceable, precise and reasonable in all other respects” as set out in Paragraph 206 of the National Planning Policy Framework and referred to as the ‘six tests’ (Department for Communities and Local Government 2014).

In line with good practice, having a clear purpose to provide answers to specific questions where significant environmental impacts have been identified, with an identified end date and confirmed outputs that provide statistically robust data sets;

Targeted to address significant evidence gaps or uncertainty, which are relevant to the project and can be realistically filled, as well as those species or features considered to be the most sensitive to the project impacts including those of conservation, ecological and/or economic importance;

Based, where relevant, on the best practice and outcomes of the latest review of environmental data associated with post-consent monitoring of licence conditions of offshore wind farms (MMO 2014a); and

Taking an adaptive approach, where appropriate.

## Existing and prospect data

Appendix B provides a full account of the existing benthic ecology information and datasets that have been used to inform the development of this BMP. Further descriptions of these datasets were set out in The SOWF ‘Benthic Monitoring Plan: Supporting Studies Document’ submitted to the MMO in November 2019.

## Consultation process

The approach to the development of the BMP has been presented for discussion with Natural England and the MMO as described by the consultation steps outlined in Table 1-4.

SOWFL has considered all consultation comments received and has incorporated them into the BMP where appropriate.

Table 1-4 Consultation process for benthic monitoring proposals.

| Consultation | Reference | Date | Document(S) |
| --- | --- | --- | --- |
| SOWFL (2019b) Sofia Offshore Wind Farm: Sofia Offshore Wind Farm: Project Update Meeting with Natural England: 20th August 2019: Benthic Note (no comments requested) | SOWFL and Ocean Ecology Ltd. | 20 August 2019 | 003235279-01 |
| Workshop with Natural England to discuss site investigation, additional characterisation surveys and development of the BMP (Meeting minutes) | - | 20 August 2019 | 003243288-01 (meeting minutes) 003195080-01 (presentation) |
| SOWFL (2019c) Sofia Offshore Wind Farm: Benthic Monitoring Plan: Supporting Studies (comments requested) Response from Natural England received Response from MMO received | SOWFL and Ocean Ecology Ltd. Natural England MMO | 20 November 2019 20 December 2019 10 January 2020 | 003097005-01 DAS/ USD 3386 303525 DCO/2013/00011 |
| Meeting with Natural England, Centre for Environment Fisheries and Aquaculture Science (Cefas) and the MMO to discuss shortlisted monitoring options (final comments awaited from Natural England to agree notes) | - | 25 February 2020 | 003527426-01 (meeting minutes) 003466055-01 (presentation) |
| SOWFL submission of mapping illustrating characterisation and vibrocore sampling locations with supporting justification. Response from Natural England received | - | 16 March 2020 23 April 2020 | - 315066/5849 |
| SOWFL response to Natural England queries and further information to support characterisation sampling locations. Response from Natural England received | - | 29 April 2020 15 May 2020 | 003591923-01 315066/5849 |
| Ongoing discussions with Senckenberg Research Institute (SRI) regarding monitoring | - | January and April 2020. Ongoing discussions. | - |
| SOWFL submission of Benthic Monitoring Plan (comments requested) Response from Natural England received | SOWFL and Ocean Ecology Ltd. Natural England CEFAS | 09 September 2020 07 October 2020 13 October 2020 | - 315066/5849 |
| SOWFL submission of updated Benthic Monitoring Plan including nearshore sampling design rationale (comments requested) Sign off following consultation with NE and Cefas | SOWFL and Ocean Ecology Ltd. MMO | 01 March 2021 09 March 2021 | 003552655-04 DCO/2013/00011 |

# project description

## overview

The SOWF site is located approximately 165 km offshore on the shallow central area of the North Sea known as the Dogger Bank with the export cable landfall in an area between Redcar and Marske-by-the-Sea (Figure 1). Water depths in the array area are between 21 m and 37 m and water depths along the export cable corridor are up to 82 m.

In the former Dogger Bank Zone, there are three other consented projects that neighbor the SOWF. These are Dogger Bank A (DBA), Dogger Bank B (DBB) and Dogger Bank C (DBC) all of which are jointly owned by SSE and Equinor. These are located 2.3 nm south west, 3.3 nm west and 2.7 nm east of SOWF, respectively.

The project array layout shown in Figure 2-1  comprising 100 wind turbines has been approved by the Maritime and Coastguard Agency and Trinity House.

Figure 2-1 The location and project layout of the Sofia Offshore Wind Farm (SOWF).

SOWF has a generating capacity of up to 1.4 gigawatts (“GW”). The footprint of the layout area is approximately 593 km². Offshore, the DCO (as amended in March 2019) allows for:

  - Offshore wind turbine generating station with a gross electrical output capacity of up to 1.4 gigawatts;

  - Up to 200 wind turbines and supporting tower structures;

  - Wind turbine foundations and associated support and access structures;

  - One offshore converter platform (OCP), and associated foundations;

  - Up to four offshore collector platforms, and associated foundations*;

  - Up to two offshore accommodation or helicopter platform(s) for operations and maintenance activities, and associated foundations*;

  - Subsea inter-array cables between the wind turbines; between wind turbines and offshore collector platforms; between wind turbines and offshore converter platform; linking to meteorological stations and accommodation platforms**;

  - Subsea inter-platform cables: between offshore collector platforms; between offshore collector platforms and the High Voltage Direct Current (HVDC) offshore converter platform**.

  - Offshore export cable systems, carrying power from the offshore HVDC converter platform to the landfall(s);

  - Crossing structures at the points where project cables cross existing subsea cables and pipelines or other Dogger Bank project cables;

  - Up to five offshore meteorological monitoring stations. This is in addition to the two meteorological stations which were subject to an earlier and separate consent application and installed in 2013;

  - Protection against scour and subsea foundation damage;

  - Seabed preparation measures;

  - Cable protection measures; and

  - Up to ten vessel mooring buoys.

*	Since the project was consented, the detailed design has been ongoing and the project currently comprises one HVDC OCP. There will be no offshore collector platforms, offshore accommodation or helicopter platform platforms, and no meteorological masts.

** The wind turbine generators will connect to each other and then directly, via the HVAC (High Voltage Alternating Current) subsea array cables, to the HVDC offshore converter platform.

## Ownership of generation and transmission assets

SOWFL will consent, design, procure, construct and commission the offshore generation system and the transmission system. Following commissioning, SOWFL (the Wind Farm Operator (“WFO”)) will transfer the assets to a licensed Offshore Transmission Owner (“OFTO”). SOWFL will retain ownership of the offshore generation system.

The offshore boundary of ownership between SOWFL and the OFTO occurs at the high-voltage and low-voltage interface between the generation system and the transmission system and is known as the Grid Entry Point (“GEP”).

Table 2-1 and Figure 2-2 shows the breakdown in asset ownership and boundary locations between the generation system and the transmission system.

Table 2-1 Asset ownership of the Wind Farm Operator and Offshore Transmission Owner.

| Non OFTO (Generation) | OFTO (Transmission) |
| --- | --- |
| - 100 wind turbines and supporting tower structures - wind turbine foundations and associated support and access structures - subsea inter-array cables (950km maximum) - array cable protection measures (where necessary) - the 66kV switchgear, SCADA systems and the control and protection panels located at the offshore converter platform (OCP) | - 400 kV bay at the National Grid Lackenby substation - 400 kV cables linking the onshore converter station (OCS) to the National Grid substation - One OCS - 320kV export cables linking the OCS to the OCP (~230km offshore, ~7km onshore), export cable protection measures where necessary. - One OCP, comprising of the HVDC converter system, converter transformers, 66kV AC and HVDC switchgear, auxiliary cooling system, diesel generators, control systems, battery DC (Direct Current) and UPS (Uninterruptable Power Supply) systems. - OCP foundations and protection against scour and subsea foundation damage (where necessary) |

Figure 2-2 Diagram showing the Grid Entry Point between the Wind Farm Operator and Offshore Transmission Owner systems.

# Development of the benthic Monitoring Plan

## Introduction

This section outlines how the BMP has been developed following a review of the existing and prospect data (Appendix G); the findings of the Environmental Impact Assessment (EIA) as reported in the ES, and the Habitats Regulation Assessment (HRA) for the project; the proposals within the IPMP; a review of the background characterisation information (SOWFL Benthic Note and Benthic Monitoring Plan: Supporting Studies documents) and discussions with the Senckenberg Research Institute (SRI), the MMO, Cefas and Natural England. A full list of associated documents that have been reviewed as part of the development relevant to this BMP is provided as Appendix H.

## findings of the environmental statement

Studies to date have identified that BHoCEEI may be present within the SOWF array and cable corridor areas (additional to the known Dogger Bank SAC sandbank feature). Key potential impacts on these habitats have been assessed for all stages of the SOWF development as part of the original EIA. Temporary disturbance of existing habitats and increases in suspended sediment levels and subsequent deposition of sediment are predicted during the construction phase, with some permanent loss of habitat where infrastructure is fixed in place during operation. The EIA established that these impacts will not be significant for SOWF nor when considering in combination of effects of SOWF and DBWF combined. This conclusion was based on the understanding that the areas of existing seabed habitats that will be subject to temporary disturbance and/or permanent loss represented only a small proportion of similar seabed habitats in the wider region. In addition, the habitats to be affected are generally considered to exhibit low sensitivity and high recoverability to the majority of effects predicted.  Furthermore, these conclusions were based on the worst-case scenario of installation of 200 WTGs, double of what is now proposed. A full description of the likely effects and their significance is given in Appendix G.

As set out in Section 1.4, the IPMP identifies that monitoring should be focused on Annex I habitat (cobble reef), with appropriate consideration given to monitoring the presence of any INNS and wider community type/structure, to identify changes that may affect site integrity.  However SOWFL will give consideration to all Annex I habitat. As such, the updated habitat map based on the 2020 SI data will be used to identify the presence and extent of reef habitat that will be potentially affected by the construction works. From preliminary evaluation of the SI data sets, the presence of biogenic reef (potentially S. spinulosa) and geogenic reef have been identified as well as peat and clay exposures. These habitats will also therefore be mapped (if found to be present in areas affected by construction) and monitored.

In addition, in order to examine the effects of SOWF, particularly scour on the Dogger Bank SAC Annex I sandbank feature, a selected number of infrastructure locations with the greatest potential for scour will be monitored. This will ensure that monitoring required under the provisions of the SOWFL Geophysical (Bathymetrical) Monitoring Plan (SOWFL, 2019a) is linked to the benthic monitoring such that the effects of the construction and operation of the wind farm can be appropriately assessed.

## Findings of the Report to Inform Appropriate Assessment

The Habitat Regulations Assessment Report, Information for Appropriate Assessment Report (Forewind, 2014) predicted there would be no adverse effect on the integrity of Natura 2000 sites as a result of construction, operation and decommissioning of the offshore and onshore components of the SOWF, either alone or in-combination with other plans and projects. This conclusion was based on the findings that adverse effects on the qualifying features (i.e., habitats and species) will not occur and this was confirmed in the Secretary of State Habitats Regulation Assessment (Department of Energy and Climate Change, 2015).

## Non-material Change Applications and deemed Marine Licence Variations

In June 2018, SOWFL submitted, and was granted (in March 2019) a request to the Secretary of State for changes to be made to the DCO for SOWF.  A parallel submission was made to the MMO, and granted (in April 2019) for the same changes to dML Schedule 9 and dML Schedule 11 of the DCO.  Of relevance to benthic ecology was the request for monopiles up to a maximum 12m diameter to be used for the Offshore Converter Platform.

The Secretary of State considered the effects of the proposed changes on the conclusions reached in the original DCO granted in 2015 (Department of Energy & Climate Change, 2015). Of relevance to benthic receptors the Habitats Regulation Assessment (“HRA”) (The Department for Business, Energy and Industrial Strategy, 2019) considered the potential effects on features of the Dogger Bank SAC of the use of monopiles for the Offshore Converter Platform.

The Secretary of State considered that the changes proposed would not compromise the conclusions of the assessments undertaken to support the original consent and therefore concluded that the proposed changes would not be likely to have a significant effect on the SAC.

## Benthic monitoring note

A Benthic Monitoring Note was submitted to Natural England in advance of a workshop held on 20th August 2019 (SOWFL, 2019c).  This Note provided an overview of the site investigations planned for 2020 and the approach to benthic monitoring and development of the BMP.  No formal comments were requested on this Note, but minutes of the meeting were agreed and issued in October 2019.

## BMP: Supporting Studies document

Following the workshop mentioned above and a review of the minutes of the meeting, SOWFL prepared a BMP: Supporting Studies document and issued it in November 2019 to the MMO and Natural England (SOWFL, 2019b).  This Supporting Studies document set out a) the existing benthic ecology position b) the additional data to be collected during the summer 2020 SI survey programme and c) the planned approach to the development of the BMP. Natural England and the MMO (incorporating advice from Cefas) provided discretionary advice (on 20th December 2019 and 10th January 2020 respectively) which highlighted a number of concerns namely with regards to the number of ground-truthing stations to be visited during the summer 2020 SI survey programme and the limitations of the resulting habitat mapping. Responses to these comments were discussed during a teleconference held with the MMO, Cefas and Natural England on 25th February 2020. This provided an update on the ground-truthing sampling to be conducted during the summer 2020 SI survey programme to include the collection of approximately 100 additional surface sediment samples along the export cable route and array area for subsequent Particle Size Distribution (PSD) analysis.

## Characterisation sampling mapping

SOWFL provided the locations and rationale for the 25 benthic grab sample sites which formed part of the 2020 ground truthing survey (see Appendix B) to the MMO, Cefas and Natural England via an online mapping application in April and May 2020. Details of the additional 100 sediment sample locations collected during the 2020 SI, which further informed the updating of the habitat map for the area, was subsequently provided to the MMO, Natural England and Cefas via the online mapping application. Natural England acknowledged SOWFL’s comments regarding the quantity of samples but, remained of the view that further benthic samples within the Dogger Bank SAC would be beneficial. As such, additional characterisation data for the SAC were collected during summer 2020 as part of ongoing long-term research programmes (see Section 3.8 and Appendix B).

## Engagement with senckenberg research institute

As set out in Appendix B, Senckenberg Research Institute (SRI) have been conducting surveys on the Dogger Bank since 1991.  SOWFL and their benthic ecology advisors, Ocean Ecology Ltd., have been engaging with SRI since 2019 to understand ongoing research surveys being undertaken on the Dogger Bank and to consider opportunities for collaboration which has so far included the collection of 25 additional grab samples across the Dogger Bank SAC during summer 2020 as a means of supplementing the sampling undertaken as part of the SI programme (see Section 3.8 and Appendix B).

Since completion of the 2020 survey campaigns, SOWL and SRI have held further discussions and have agreed to further collaborate during the 2021 long-term monitoring survey and as part of a focused research study to be undertaken by SRI aimed specifically at studying the potential regional scale impacts of SOWF and DBW A, B and C on the wider Dogger Bank. This additional study (known as ‘REWIND’) will essentially act to provide an increased sampling density complementing SRI’s long-term monitoring conducted annually since 1991. SOWL have agreed to assist SRI in the field during the long-term and REWIND cruises in 2021 in exchange for grab samples to be taken at each of the long-term and REWIND sampling stations equating to an additional 82 sampling stations (37 long-term and 45 REWIND). SOWL have agreed to undertake macrobenthic and PSD analysis of all 82 sample which will provide a robust dataset for assessing potential broad-scale regional impacts of SOWF and DBW A, B and C on the Dogger Bank SAC sandbank feature.

SOWF and SRI have worked collaboratively to rationalise the SOWF BMP and SRI sampling designs. This process has involved micrositing of the existing SRI long-term monitoring stations to ensure there will be no future conflicts with any of the planned OWF infrastructure across Dogger Bank whilst also microsting the planned REWIND sampling stations to optimize the design ensuring maximum geographic coverage whilst also avoiding unnecessary repetition of sampling in similar areas, impact zones and/or habitats. The SRI sampling stations have also been micro-stied to ensure they were not positioned within areas of predicted ‘nearfield’ impacts of infrastructure for which the targeted SOWF BMP sampling set out in this document will monitor. The sampling design for the SRI research cruises are set out in Appendix K accompanied by a rationale for micrositing of each sampling station.

## relevant guidance documents

The MMO post-consent monitoring (PCM) review (MMO, 2014a) and corresponding MMO recommendations (MMO, 2014b) noted that there is limited value in broad-scale generic benthic monitoring to assess potential impacts of offshore windfarm developments and that efforts are better focused on key areas or receptors to be impacted. This BMP has therefore been focused on monitoring BHoCEEI that are most likely to be impacted by the construction and operation of the SOWF. This will follow approaches adopted at other UK offshore wind farm sites in the North Sea including Triton Knoll Offshore Wind Farm (RWE). The approaches proposed will also allow for assessment of consequential impacts of colonisation of the project infrastructure to adjacent peripheral seabed areas helping to establish whether ‘feeding halo’ effects or similar impacts occur (MMO, 2014a).

# Aproach To benthic Monitoring

## Features of focus

Based on the current understanding of the benthic habitats found across the SOWF array and cable corridor areas (see Appendix B), the SOWF benthic monitoring will involve collection of underwater imagery and grab samples for macrobenthic and PSD analysis.  Given the scale of the SOWF project, length of the cable corridor and effects predicted as part of the EIA and HRA, this sampling will be targeted only on Features of Focus (FoF) that are likely to be affected by the construction and operation of SOWF and not all of those present within the wider Order Limits.

As discussed in Section 1.3 and presented in Table 1-3, the SOWF benthic monitoring requirements fall into two main categories - those relating to BHoCEEI (including sediment and reef habitats) and other monitoring targeting particular receptors or focusing on potential impacts that were identified in the ES (i.e. INNS). This section sets out the proposed approach for monitoring each of these key FoF.

## Feature of Focus i: sediments

### Objective

The main objective of the monitoring focused on assessing the key sediment habitats will be to characterise and identify, where possible, any localised effects of the project infrastructure (foundations and cables) on key sediment habitats and the infaunal and epibiotal communities (including demersal fish) found within the Dogger Bank SAC as a means to validate the key predictions made in the ES. It will also allow for the identification of non-native species to soft substrate habitats e.g. rock/hard substrate specialists which would not otherwise be present.

### Survey design

The habitat mapping based on all available data across the area (see Appendix B and L) has been considered alongside the outputs of the scour assessment to ensure a stratified sampling approach for monitoring the key sediment habitats within the Dogger Bank SAC. The approach will survey a sample of 17 infrastructure locations (foundation structures including the OCP jacket and any associated scour processes or scour protection, export cables and array cables) representative of the varying depths, habitats and levels of predicted scour across the array area and portion of the Export Cable Route (ECR) that overlaps the Dogger Bank SAC. The full rationale for positioning of all proposed sampling stations is provided in Annex 1.

Sampling locations have been selected to represent the different sediment habitats identified across the Annex I sandbank feature (see Appendix L) with the aim of examining a range infaunal, epifaunal and demersal fish communities associated with the feature. No sediment sampling is proposed for the section of the export cable corridor that lies outside the Dogger Bank SAC, as the characterisation work undertaken at the ES stage confirmed that the seabed habitats and communities are similar to those within the Dogger Bank SAC and no significant impacts were predicted.

Topographical impacts of seabed infrastructure and associated scour on the Annex I sandbank feature will also be monitored as part of the approved Geophysical (Bathymetrical) Monitoring Plan (SOWFL, 2019a). For pre-construction, this involved collection of MBES and SSS data at all generation (WTG and inter-array cables) and transmission (OCP and export cables) infrastructure locations including buffer zones in 2020. MBES and SSS data will also be collected during post-construction geophysical monitoring surveys at the same subset of infrastructure locations (within the Order Limits) that will be targeted with underwater imagery and grab sample collection as part of this BMP. All geophysical monitoring data will be collected to International Hydrographic Order (IHO) Order 1A standards.

Figure 4-1 Proposed arrangement of benthic sampling at each selected WTG and the OCP location. Note that BRUV sampling will only take place at a subset of three WTG locations. Also shown on the left is the modelled tidal flow axis for the array area showing the predominant NW to SE direction.

### Approach

The survey approach will adopt a cruciform sample alignment, focusing on nearfield impacts inclusive of scour. A sample of eight WTG locations, the OCP jacket, four inter-array cables and four locations along the export cable route where key habitats cannot be avoided will be selected.

Infaunal and Sessile Communities

At each of the foundation locations, a cruciform pattern of sampling stations will be arranged with eight stations to be sampled at each location. Buffers were placed around the proposed locations at 100 m, 200 m and 300 m. Eight stations were placed around the proposed WTG locations in a cruciform arrangement. The proposed cruciforms consist of ‘long arms’ aligned with the NW to SE tidal axis extending 300 m from each WTG location and ‘short arms’ aligned at right angles to the ‘long arms’ extending 100 m from each WTG location. Sampling stations were positioned at the 100 m, 200 m and 300 m points along each of the ‘long arms’ and at the 100 m point of the ‘short arms’ (see Figure 4-1). The ‘long arms’ were aligned to the dominant tidal flow (NW to SE). Stations were arranged to avoid interaction with the inter-array cables. At each of the inter-array cable and export cable corridor locations, stations were arranged 15 m either side of the cable route (a total of two stations per cable location).

Twelve control stations will also be sampled. These will be positioned in areas representative of the corresponding key sediment habitats to be targeted by the foundation and export cable locations within the SAC boundary but outside the main tidal excursion of the SOWF infrastructure locations and away from any other nearby impacting activities e.g. oil and gas developments (if present) and the other planned Dogger Bank offshore wind farm sites. Where possible, the control stations have been located to align with sampling positions visited during JNCC’s sampling campaign of Dogger Bank in 2015 to provide long-term temporal context.

Sampling at each station will firstly involve the collection of seabed imagery in transect format running from as close as possible to the WTG locations (and/or edge of scour protection if installed at particular turbine) out to 150 m distance in one direction (‘short arms) and 300 m in the other (long arms). This will include collection of continuous video and high-resolution stills at 5-10 m intervals. This will be followed by the collection of triplicate grab samples at each of the eight sampling stations. All seabed imagery will be collected in line with the latest JNCC/NMBAQC guidelines (Hitchin et al., 2015). All grab samples will be collected using a 0.1m2 mini-Hamon grab in line with the protocols set out in the latest version of Regional Seabed Monitoring Programme (RSMP) protocol (Cooper & Mason 2020) and will undergo PSD and macrobenthic analysis at a laboratory participating in the NMBAQC scheme in line with the latest NMBAQC Best Practice Guidance (Mason 2016). Biomass will be measured as blotted wet weight in grams to at least 4 decimal places for all countable taxa (i.e. at species level where possible). In addition to standard Quality Control procedures, macrobenthic and PSD sample processing will be subject to external quality control checks by an independent, competent benthic laboratory participant in the NMBAQC scheme as per the RSMP protocol (Cooper & Mason 2020).

Further monitoring information relating to the infaunal and sessile communities associated with the wider Annex I sandbank sediments will be collected during the annual investigations of the mega-epibenthos on the Dogger Bank conducted by SRI (see Section 3.8, 4.2.15, Appendix B and Appendix K) and incorporated into the pre- and post-construction monitoring. The combination of the infrastructure focused and wider sampling conducted by SRI will ensure both localised and regional broad scale impacts on the infaunal and sessile communities associated with the Annex I sandbank feature are appropriately monitored.

- Motile Epifauna and Demersal Fish Communities

At a subset of three of the WTG locations (both pre- and post-construction), baited remote underwater video (BRUV) systems will be deployed to the seabed to assess the motile epifaunal and demersal fish species. BRUV systems will also be deployed at the OCP position, a subset of one of the inter-array cables (one side only), two of the export cable route locations (one side only) and at five of the control locations. The methods used will be based on recent developments in the use of BRUVs as monitoring tools in low visibility environments around offshore renewable infrastructure (Griffin et al., 2016; Jones et al., 2020; Unsworth et al., 2014) and will provide a low cost and non-extractive method for assessing motile epibiota and demersal fish species in the vicinity of the infrastructure locations that would otherwise be missed through use of drop-down video transects alone. This information will be considered alongside the data collected during the annual investigations of the mega-epibenthos on the Dogger Bank conducted by SRI. This includes sampling using a 2 m beam trawl and a ring dredge at 37 long-term monitoring stations located across the Dogger Bank SAC including several within the SOWF array and export cable corridor areas (see Appendix B). The combination of the BRUV sampling and consideration of the SRI datasets will ensure both localised and far field impacts on the epibiotic and demersal fish communities associated with the Annex I sandbank feature are appropriately monitored.

### Cumulative Impacts

SOWF is located within the Dogger Bank SAC, as are the Dogger Bank A, B and C sites (Figure 1). Despite all four projects being consented at similar times and following broadly similar development timelines, based on discussions with the Dogger Bank projects, the start dates of offshore construction activities are not expected to be the same.  Whilst certain construction activities may overlap for some of the projects, construction activities in the array area for all four projects occurring at the same time is not anticipated.

Chapter 9 (Marine Physical Processes) of the ES (Forewind, 2014) stated that the potential for the accumulation of thick ‘sequences’ of sediment due to plume interaction (and associated deposition) was considered to be low meaning a negligible cumulative impact on marine ecological receptors was predicted. This was for a scenario that considered the concurrent build of all four of the projects within the Dogger Bank SAC, although the known schedules of offshore construction now indicate that it is highly unlikely that this will be the case. Longer periods of time between ‘disturbance events’ will further reduce any potential for cumulative impacts to arise from suspended sediment and deposition.

Since cumulative impacts were not predicted in the ES and the anticipated construction timeframes further reduce any potential for cumulative impacts, no further sampling is proposed as part of the SOWFL specific monitoring to assess regional scale cumulative impacts.

SOWFL have however agreed to collaborate with SRI to collect an additional 82 0.1 m2 Hamon grab samples across the Dogger Bank SAC in summer 2021 which will be used alongside the ring dredge and beam trawl data to be collected as part of SRI’s long-term monitoring to act as a baseline against which future monitoring data can be compared to assess for potential broad-scale regional impacts on the Annex I sand bank feature of the Dogger Bank SAC.

Combined with the SRI long-term data set, this newly acquired data alongside the project specific sampling data will help to differentiate any changes attributable to the development of SOWF from the other dominant anthropogenic activities across the Dogger Bank SAC (e.g. bottom trawling). This will enable more accurate conclusions to be drawn with respect to the reasons for any changes in the benthic faunal communities across Dogger Bank over a timeframe that would not normally be possible for a monitoring programme for a standalone offshore wind farm project.

## Feature of Focus II: Biogenic/geogenic reef

### Objective

The main objective of the monitoring focused on assessing biogenic/geogenic reef habitats will be to characterise and identify, where possible, any localised effects of the project infrastructure (foundations and cables) on directly affected habitats as a means to validate the predictions made in the ES.

### Approach

A targeted sampling approach has been adopted to ensure that representative examples of each of the habitat(s) confirmed to be present during the 2020 SI survey programme within the proposed construction areas are suitably monitored. This will involve the collection of seabed imagery in the form of continuous video and high-resolution stills every 5-10 m along transects positioned to determine the extent of the habitats and ground-truth interpretation of the geophysical survey data to be collected as part of the Geophysical (Bathymetrical) Monitoring Plan (SOWFL, 2019a). All seabed imagery will be collected in line with the latest JNCC/NMBAQC guidelines (Hitchin et al., 2015). Detailed analysis of the still images will also provide a means of quantitatively assessing the other relevant monitoring indicators for each habitat identified (e.g. elevation and patchiness for S. spinulosa reefs if encountered).

Due to the scale of the SOWF array area and length of the export cable route, SOWFL have proposed that a number of representative Areas of Focus (AoF) are targeted for monitoring despite multiple areas of reef habitat being located within the proposed construction area. This means that a subset of the reef habitats identified within the proposed construction area during the 2020 SI campaign have been selected to act as proxies for the other reef areas not to be sampled. Appendix I provides the final sampling design along with the rationale for the nearshore transects out to KP 10 whilst Appendix J provides the same for offshore areas beyond KP 10.

All seabed imagery analysis will be undertaken in consideration of the latest NMBAQC/JNCC epibiota remote monitoring interpretation guidelines (Turner et al. 2016) using annotation software (e.g. the BIIGLE annotation platform (Langenkämper et al. 2017)) to fully record counts and percentage coverage of all epibenthic taxa visible. EUNIS classifications will subsequently be assigned to all stills in consideration of the most recent JNCC guidance on assigning benthic biotopes (Parry 2019). When S. spinulosa and/or stony reef habitats are observed, the still images will also undergo a ‘reefiness’ assessment in line with Gubbay (2007) and Irving (2009). Ten percent of the stills will be subject to internal Quality Assurance (QA). Results will be provided in a excel spreadsheet complying with MEDIN data standards, with the most up to date names checked against the World Register of Marine Species (WoRMS) database.

## Feature of Focus iII: Invasive non-native species

### Objective

The objective of this monitoring is to identify any INNS in the wind farm site and any effects on the wider community type/structure as a result of wind farm development, to identify changes that may affect site integrity.

Monitoring for INNS relates to the post-construction period only. Prior to any survey works, a comprehensive review of all available literature and data on INNS in the Dogger Bank region will be undertaken including information available from the characterisation surveys that informed the ES, long term monitoring data collected by SRI, statutory monitoring of the Dogger Bank SAC undertaken by JNCC and any other relevant surveys undertaken in the region (see Appendix B).

Infaunal, sessile and motile epibiotia (including demersal fish) data collected as part of the sediment and reef monitoring set out in Sections 4.2 and 4.3 respectively will also be reviewed to establish if any INNS are identified during the pre-construction stages. This will include consideration of the outputs of the geophysical surveys with respect to wider community type/structure, in line with the MMO (2014a, 2014b) focus on the establishment of mature epibenthic communities on foundations and scour protection within the context of studies of the wider operational area.

To supplement the data to be collected during the sediment and reef post-construction monitoring data, all foundation structures (including the OCP jacket), scour protection and export cable/inter-array protection will also be surveyed to investigate the degree of colonisation and epibenthic community composition with a focus on confirming the presence/absence of any INNS. Note that only export cable and inter-array cable locations within the Dogger Bank SAC will be monitored if protection is installed). At each location, underwater imagery of the structure will be collected from the surface down to the seabed. This will include extension of the data collection out across the scour protection at each location and out across the sediment substrate at the OCP foundations where it is currently proposed that no scour protection will be installed. The data collected will undergo detailed analysis including the identification and enumeration of all epibiota observed on the structures (i.e. not just limited to INNS). Photogrammetry techniques will also be used to estimate epibiotic biomass on each structure and / or associated scour protection. This will allow for suitable statistical analysis to be undertaken on both abundance data and biomass estimates between monitoring periods which will also be considered when assessing for potential impacts of colonization of hard substrates by INNS on the surrounding and wider sedimentary benthic communities,

It should be noted that all infaunal, sessile and motile epibiotia (including demersal fish) data collected as part of the sediment, reef and INNS monitoring set out in sections 4.2, 4.3 and 4.4.4 will not only be reviewed to establish the presence/absence of INNS but will also undergo detailed analysis to identify all taxa present (i.e. including indigenous species). These datasets will undergo suitable statistical analysis to test for potential impacts on the composition of communities associated with the Dogger Bank SAC sandbank feature. This will include investigation of near and far field impacts and comparison to corresponding data from control stations.

## Monitoring Schedule

It is proposed that the pre-construction sediment (FoF I) and reef (FoF II) sampling is undertaken simultaneously in 2021 with the full results submitted to the MMO approximately 6 months post completion of the surveys. Due to UXO clearance activities planned for Q2 2021 in the nearshore area (out to KP10), it is proposed that the pre-construction survey programme is undertaken during separate nearshore and offshore phases in Q1/Q2 2021 and Q2/Q3 2021 respectively. The SRI long-term monitoring and REWIND sampling will be undertaken during two separate research cruises in July/August 2021.

Post-construction surveys (including the targeted INNS survey (FoF III)) will be undertaken approximately 1-year post commissioning to allow sufficient time for initial recolonisation of habitats (anticipated to be 2026/27). This timeline for further survey has been proposed to allow equilibrium to be reached following construction and to tie in with operational bathymetrical monitoring that will be undertaken as part of the Geophysical (Bathymetrical) Monitoring Plan (SOWFL, 2019a). Further post-construction surveys for all FoF will be undertaken 3, 5 and 10 years post-commissioning as well as prior to decommissioning (as part of studies required for the marine licence application to decommission the wind farm). A substantive review will be undertaken following completion of the year 5 monitoring survey that will consider all pre- and post-construction monitoring data in the context of the predictions made in the EIA and the benthic monitoring objectives for the Dogger Bank SAC set out by NE and JNCC (Appendix A). At this point, the requirement for further post-construction benthic monitoring for each FoF will be discussed with the MMO and its advisors.

Should monitoring continue until year 10, the final monitoring report will also include a thorough review of the need for future monitoring beyond                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                           10 years. This will consider recovery of the SAC in areas affected by the project in the context of the monitoring objectives, the conclusions of the ES and HRA and in light of other external pressures (such as fishing) that will also be impacting the recovery of the SAC.

## Reporting

Following completion of the pre-construction survey and associated analysis, a detailed benthic technical report will be prepared following the structure set out in Table 4-1. This report will clearly set out the baseline conditions against which any post construction survey data will be compared (if required). The requirements for post construction monitoring will be presented in the pre-construction report.  The pre-construction report will be submitted to the MMO within 6 months of completion of the survey.

Any post-construction reporting required will follow a similar format and include an assessment of the effects of the project on the FoF and will identify any requirements for ongoing monitoring. The post-construction reports will also be submitted to the MMO within 6 months of completion of the surveys.

Table 4-1 Outline of pre-construction benthic baseline reporting structure.

| Section | Description |
| --- | --- |
| Introduction | - Project background - Existing environment - Aims and objectives |
| Methods | - Sampling design and rationale |
|  | - Field methods |
|  | - Laboratory methods |
|  | - Statistical analysis |
| Results | - Summary of progress |
|  | - Sediment analysis and mapping |
|  | - Macrobenthic analysis and mapping |
|  | - Seabed imagery analysis and mapping |
|  | - Mapping of benthic habitats of BHoCEEI |
|  | - Mapping of non-native species occurrences |
|  | - Comparisons to historic datasets and existing mapping |
| Discussion | - Contextualisation of results |
|  | - Limitations of study |
|  | - Conclusion |
| References | - |
| Appendices | - Sampling logs |
|  | - Data matrices |
|  | - Grab sample photographs |
|  | - Seabed imagery examples |

# References

Cooper, K., Mason, C. (2020). Regional Seabed Monitoring Programme (RSMP) Protocol for Sample Collection and Processing (Version 8.0). Cefas.

Department for Communities and Local Government. 2014. Available at: http://planningguidance.planningportal.gov.uk/blog/guidance/use-of-planning-conditions/application-of-the-six-tests-in-nppf-policy/ [Accessed 24 April 2014].

Diesing, M., Ware, S., Foster-Smith, R., Stewart, H., Long, D., Vanstaen, K., Forster, R. & Morando, A. (2009). Understanding the marine environment – seabed habitat investigations of the Dogger Bank offshore draft SAC. Joint Nature Conservation Committee, Peterborough. JNCC Report No. 429. JNCC, Peterborough.

Forewind (2014) Dogger Bank Teesside A & B Environmental Statement – Marine and Intertidal Ecology and relevant appendices (Appendix 9A Marine Physical Processes Assessment of Effects Technical Report; Appendix 12A Tranche B and Export Cable Corridor Benthic Survey Report; Appendix 12B Tranche A Benthic Survey Report; Appendix 12C Intertidal Benthic Survey Report; Ap Appendix 12D Tranche B Habitat Mapping Report; Appendix 12E Value Ecological Receptor (VER) Tables; and Appendix 12F Disposal Site Characterisation Document.

Forewind (2015) Dogger Bank Deadline IX Appendix 4 Offshore In Principle Monitoring Plan

Griffin, R. ., Robinson, G. ., West, A., Gloyne-Phillips, I. ., & Unsworth, R. F. . (2016). Assessing Fish and Motile Fauna around Offshore Windfarms Using Stereo Baited Video. PLOS ONE, 11(3), e0149701. https://doi.org/10.1371/journal.pone.0149701

Gubbay S (2007) Defining and managing Sabellaria spinulosa reefs: Report of an inter-agency workshop 1-2 May, 2007. JNCC Rep No405 44:22

Hitchin, R., Turner, J., & Verling, E. (2015). Epibiota Remote Monitoring from Digital Imagery: Operational Guidelines. NMBAQC and JNCC. http://www.nmbaqcs.org/media/1591/epibiota_operational_guidelines_final.pdf

Irving R (2009) The identification of the main characteristics of stony reef habitats under the Habitats Directive. Summary report of an inter-agency workshop 26-27 March 2008. JNCC Rep No 432:44.

JNCC (2014) Dogger Bank SCI 2014 Monitoring R&D Survey Report. 162 pp.

JNCC (2015) Dogger Bank SCI Monitoring Survey – CEND 10/14 Cruise Report. 218 pp.

JNCC (2018) Supplementary Advice on Conservation Objectives for Dogger Bank Special Area of Conservation. 28 pp.

Jones, R. E., Griffin, R. A., Januchowski-Hartley, S. R., & Unsworth, R. K. F. (2020). The influence of bait on remote underwater video observations in shallow-water coastal environments associated with the North-Eastern atlantic. PeerJ, 8(August). https://doi.org/10.7717/peerj.9744

Langenkämper D, Zurowietz M, Schoening T, Nattkemper TW (2017) BIIGLE 2.0 - Browsing and Annotating Large Marine Image Collections. Front Mar Sci 4:83.

Mason C (2016) NMBAQC’s Best Practice Guidance - Particle Size Analysis (PSA) for Supporting Biological Analysis.

MMO (2014a). Review of post-consent offshore wind farm monitoring data associated with licence conditions. A report produced for the Marine Management Organisation, pp 194. MMO Project No: 1031. ISBN: 978-1-909452-24-4.

MMO (2014b). The Marine Management Organisation (MMO) Response to an Independent Review of Environmental Data Collected at UK and European Offshore Wind Farm and 22 Recommendations for Future Post-Consent Monitoring. Available at: https://www.gov.uk/government/uploads/system/uploads/attachment_data/file/389392/MMO_response_to_independent_OWF_PCM.pdf.

Parry ME V (2019) Guidance on Assigning Benthic Biotopes using EUNIS or the Marine Habitat Classification of Britain and Ireland.

SOWFL (2019a) Detailed Construction and Monitoring Programme: Geophysical (Bathymetrical) Monitoring Plan. Version 04.

SOWFL (2019b) Sofia Offshore Wind Farm: Benthic Monitoring Plan: Supporting Studies

SOWFL (2019c) Sofia Offshore Wind Farm: Sofia Offshore Wind Farm: Project Update Meeting with Natural England: 20th August 2019: Benthic Note

Turner J.A., Hitchin R., Verling E., Rein H. van. (2016) Epibiota remote monitoring from digital imagery: Interpretation guidelines. JNCC

Unsworth, R. K. F., Peters, J. R., McCloskey, R. M., & Hinder, S. L. (2014). Optimising stereo baited underwater video for sampling fish and invertebrates in temperate coastal habitats. Estuarine, Coastal and Shelf Science, 150(PB), 281–287. https://doi.org/10.1016/j.ecss.2014.03.020

UK BAP (2008a). UK Biodiversity Action Plan; Priority Habitat Descriptions. BRIG (ed. Ant Maddock) 2008 - Peat and Clay Exposures with Piddocks. Updated 2016.

UK BAP (2008b). UK Biodiversity Action Plan; Priority Habitat Descriptions. BRIG (ed. Ant Maddock) 2008 - Peat and Clay Exposures with Piddocks. Updated 2016.

Worsfold A.T., Hall D. (2010) Guidelines for processing marine macrobenthic invertebrate samples: a Processing Requirements Protocol.

Benthic monitoring objectives for the Dogger Bank SAC set out by Natural England

EXISTING DATA USED IN THE DEVELOPMENT OF THE BMP.

Existing Data

- There exists a large volume of information relating to the benthic habitats and their associated communities located across the area of seabed where construction works are proposed. This has been collected during the site characterisation surveys of the wider Dogger Bank Zone in 2010-2011, statutory monitoring of the Dogger Bank Special Area of Conservation (SAC) by the Joint Nature Conservation Committee (JNCC) and long-term monitoring undertaken by the Senckenberg Research Institute (SRI) and others.

- Further up to date characterisation data has been collected by SOWFL as part of the geotechnical and geophysical site investigation (SI) campaign undertaken throughout spring and summer 2020. This involved collection of high-resolution acoustic data (side scan sonar (SSS) and multibeam echosounder (MBES)) along with a large volume of ground-truthing information including sediment samples and seabed imagery. This data will enable the existing habitat and biotope mapping to be updated in areas potentially affected by construction of the SOWF. It will also support the design of a robust benthic sampling array suitable for validation or otherwise of the key predictions made in the Environmental Statement (ES).

Site Characterisation 2010/2011 and validation exercise (2019)

- During the Environmental Impact Assessment (EIA) phase of the SOWF development process two comprehensive seabed mapping surveys were conducted of the wider Dogger Bank Zone (Tranche A and B) involving the collection of a large volume of geophysical and environmental data across the area. This included acquisition of SSS and MBES data for mapping seabed habitats followed by grab sampling and collection of seabed imagery for ground-truthing the interpretation of the processed geophysical data (Figure F1). The data allowed the production of a detailed biotope map for the entire zone and export cable route options (Appendix C). The biotopes were then grouped into Valued Ecological Receptor (VER) categories for the purposes of assessing the potential impacts of the construction and operational phases of the project (Appendix D). A full description of the VER categories identified during the EIA is provided in Appendix E and the predicted effects of the development and operation of SOWF on benthic habitats is outlined in Appendix F.

- Due to the age of the existing benthic survey data for the SOWF array and export cable corridor areas (7-8 years old), SOWFL conducted a Drop-Down Video (DDV) survey along specific sections of the export cable up to 12nm offshore in June 2019.  This was a means of confirming the presence/absence of potential reef habitats in a number of key Areas of Interest (AOI) including within and in the vicinity of the proposed Horizontal Directional Drill (HDD) area (see Appendix C of the BMP Supporting Studies Document for full details (SOWFL, 2019a)). The results were used to inform a preliminary cable routing workshop involving archaeological and ecological specialists and were presented to the MMO and Natural England at the project update meeting held on 20th August 2019. The results generally aligned with the habitat/biotope mapping derived from the 2010-2011 characterisation survey data and confirmed the presence of cobble and bedrock reef habitat as well as discrete patches of the Section 41 priority habitat ‘Peat and Clay Exposures’.

Historic Data Sources 1951-2008

- Comprehensive historical information regarding spatial and temporal variability in the distribution of infaunal communities present in association with the Dogger Bank habitats is provided by Kröncke (1990, 1991), Kröncke and Rachor (1992), Kröncke and Knust (1995), Wieking and Kröncke (2003, 2005) (Figure F1). As part of these studies, a number of historical time series stations/transects were established and sampled between 1951-2007. More recent survey work was conducted during April 2008 to provide data and evidence to support the recommendation of Dogger Bank to the European Commission in 2011 as a SAC. This characterisation survey incorporated both MBES survey and ground-truthing elements (Hamon grab and seabed imagery) and the results of which largely supported those reported by previous historical comparative studies (Diesing et al. 2009).

Dogger Bank SAC Monitoring 2014

- In 2014, a pilot monitoring survey at the Dogger Bank Site of Community Importance (SCI) (now SAC) was carried out collaboratively, between the Joint Nature Conservation Committee (JNCC) and Centre for Environment, Fisheries and Aquaculture Science (Cefas). The survey gathered ‘baseline’ data to help inform on the effectiveness of several proposed fishery management areas and to investigate changes in biological communities along a fishing pressure gradient. The survey involved the collection of approximately 500 macrobenthic and PSD samples, 1,500 images and 23.5 hours of video footage of the seabed across the Dogger Bank Annex I Sandbank feature. Following a request to the JNCC, the full macrobenthic, PSD and seabed imagery datasets have been made available to SOWFL to help inform the development of the BMP. These datasets have also been used alongside newly acquired geophysical survey data to help ensure geotechnical sampling locations sampled during the 2020 SI survey have not conflicted with areas likely to constitute benthic habitats of CEEI.

Long-Term Monitoring Data

- The Department of Marine Zoology at the SRI in Frankfurt am Main has been carrying out annual investigations of the mega-epibenthos on the Dogger Bank. They have sampled at 37 stations since 1991 with a 2 m beam trawl and a ring dredge and macrobenthic investigations at a number of these stations in 1991 and 2004.  SOWFL have been in discussions with SRI since 2018 in relation to the use of these long-term monitoring datasets to inform the final preconstruction baseline sampling design and for addressing concerns raised by Cefas in relation to potential regional scale cumulative impacts of the planned Sofia, Dogger Bank A, B and C offshore wind farms on the Dogger Bank SAC sandbank feature.

- Furthermore, SRI incorporated additional 0.1 m2 Hamon grab sampling at a subset of their 37 long-term monitoring stations to augment the grab samples to collected during SOWFL’s SI ground-truthing (characterisation) survey in July/August 2020 (see below). These additional samples subsequently underwent macrobenthic and PSD analysis and were used to train the predictive modelling developed to update the habitat mapping for the area.

Figure F1. Sediment (top) and seabed imagery (bottom) sampling stations visited during previous and planned surveys across the Dogger Bank zone. An interactive web map displaying this data is available here.

site investigations

- Throughout spring and summer 2020, SOWFL undertook an engineering (geophysical and geotechnical) site investigation programme comprising a series of survey campaigns across the SOWF array area and along the export cable corridor. The SI included collection of high-resolution acoustic data (Order 1A MBES and SSS) at all proposed generation and transmission infrastructure locations including survey coverage buffers as below:

Wind Turbine foundations: buffer areas of approximately 500 m by 500 m would be surveyed around each wind turbine (square area with radius 250m around the centre point of each foundation)

Array cables: buffer areas of 50 m (25 m either side of the cables) along the length of the inter-array cables.

Offshore Converter Platform: 800m radius around the centre point of the structure (1600 x 1600m square area)

Offshore Export Cables: 175m corridor

Horizontal Direction Drill (HDD) nearshore exit point: 500 X 500 area

2 Export Cables from nearshore Horizontal Directional Drill (HDD) exit point to landfall (below MLWS): 100 m corridor (25 m either side of each cable and allows for cable separation)

- Following processing, the acoustic data underwent a detailed review to identify any potential benthic habitats of CEEI that were subsequently ground-truthed during a ground-truthing survey undertaken in 2020 (see below). The final processed geophysical data and ground-truthing information was then used to develop a predictive model to update the existing habitat/biotope mapping available for the SOWF array and ECR areas (see Appendix L).

SI Investigation Ground-Truthing (characterisation) survey

- As discussed above, the targeted ground-truthing survey conducted in summer 2020 was informed by the results of the SI, full details of which are provided in the BMP Supporting Studies Document (SOWFL, 2019b). This involved the collection of grab samples at 30 stations (checked in advance via drop-down camera for presence of sensitive habitats) combined with investigation of 34 Drop Down Camera (DDC) cruciform transects to ground-truth the interpretation of the geophysical data and delineate the boundaries of any potential benthic habitats of CEEI. The resulting data has been used to train a predictive model that has been used to update the existing habitat mapping available of the area (see Appendix L) which in turn has been used to finalise the pre-construction baseline sampling locations. As part of the 2020 SI survey campaign, SOWFL also conducted a comprehensive geotechnical investigation involving cone penetration tests (CPTs) and collection of vibrocores at 124 locations along the export cable route, at a number of the proposed WTG locations and at approximately 15 inter-array cable locations (Figure F1). As a means of further ground-truthing the interpretation of the geophysical survey data, SOWF conducted PSD analysis of the surface 15 cm of the vibrocores collected. This information was then also used to further train the predictive model developed to update the existing habitat mapping for the SOWF array and ECR areas and therefore used to inform the design of the finalising the BMP sampling array.

Benthic Habitats of CEEI

- Review of the existing data have identified that several benthic habitats of CEEI exist within the SOWF Order Limits, although some are not located within the Dogger Bank SAC, nor are features of the SAC, including:

Annex I ‘sandbanks which are slightly covered by sea water all the time’ (Annex I Primary feature for site selection);

Annex I ‘stony/cobble reefs’ (component of Dogger Bank SAC but not listed as a primary feature for site selection nor identified as a qualifying feature);

Section 41 / UK BAP Priority Habitat ‘subtidal sands and gravels’;

Section 41 / UK BAP Priority Habitat ‘mud habitats in deep water’; and

Section 41 / UK BAP Priority Habitat ‘peat and clay exposures’.

- Despite being noted during the Tranche A & B surveys (2011 and 2012), the distribution of these habitats across the array and cable corridors was not mapped in detail at the time, meaning their full extent and condition within the Order Limits remains relatively unknown. A full list of habitats potentially present within the SOWF Order Limits (although potentially not within the infrastructure construction areas) is detailed in Table F1.

Table F1 Benthic habitats/biotopes of CEEI previously recorded within the SOWF Order Limits.

| Representative Habitats/Biotopes | Protection Status | Conservation Interest | Location & relationship to Dogger Bank SAC) |
| --- | --- | --- | --- |
| Circalittoral coarse sediment; circalittoral fine sand; circalittoral muddy sand; circalittoral mixed sediment) | Annex I Habitats Directive | Annex I sandbanks that are slightly covered by seawater all the time; FOCI (Subtidal sands and gravel); UK BAP priority habitat (Subtidal sands and gravels) | Array and offshore section of export cable (within SAC boundary) |
| Sabellaria spinulosa on mixed substrata | Annex I Habitats Directive | Section 41 Priority Habitat; Potentially corresponds to Annex I ‘S. spinulosa reefs’; OSPAR List of Threatened and/or Declining Species and Habitats (Region II – North Sea, Region III – Celtic Sea); Feature of Conservation Importance (FOCI) | Cable Route potentially within and outside SAC boundary. |
| Soft rock communities | None | Section 41 Priority Habitat; FOCI (Peat and Clay Exposures) | Cable Route potentially within and outside SAC boundary. |
| Seapens and burrowing megafauna in circalittoral fine mud | None | Section 41 Priority Habitat; FOCI | Array / Cable Route |
| Brittlestar beds on sublittoral mixed muddy sediments | None | Section 41 Priority Habitat (May occur within sandbanks that are slightly covered by seawater all the time) | Array / Cable Route |

invasive NON-NATIVE species

- Cefas’ OneBenthic Non-Native Species Tool provides distribution records for non-native benthic species  as identified by the UK Technical Advisory Group (UKTAG) based on large quantities of publicly available grab sample data (including the 2010/2011 site characterisation and 2014 JNCC data discussed above). Table F2 sets out a non-exhaustive list of invasive species previously recorded in the vicinity of SOWF with the potential to be found during the BMP surveys.

Table F2 Non-native benthic species previously recorded or with the potential to occur within or in the vicinity of the SOWF array and export cable corridor.

| Species | Common Name |
| --- | --- |
| Crepidula fornicata | Slipper limpet |
| Austrominius modestus | Australasian barnacle |
| Goniadella gracilis | A polychaete worm |
| Mya arenaria | Sand gaper |
| Petricolaria pholadiformis | American piddock |

Offshore In Principle Monitoring Plan and rationale of monitoring presented within this benthic Monitoring  plan

Table A1 Rationale of monitoring presented with the Benthic Monitoring Plan in relation to the Offshore In Principle Monitoring Plan.

| Potential effect | Receptor/s | Phase | Headline reason/s for monitoring | Monitoring proposed in the IPMP | Details within the IPMP | Rationale of monitoring presented within BMP |
| --- | --- | --- | --- | --- | --- | --- |
| Effects on Annex I habitats | Annex I habitats and linked receptor groups e.g. fish | Pre-construction |  Establish location and extent of benthic Annex I habitats within the order limits (outside of the Dogger Bank SCI) and validate associated predictions made in the Environmental Statement (Natural England and JNCC requirement) | Appropriate surveys (likely to be SSS/AGDS/DDV/Grab) to determine the location and reasonable extent of any benthic habitats of conservation, ecological and/or economic importance (including Annex I habitats) in whole or in part inside the area(s) within the Order limits in which it is proposed to carry out construction works. | To include a pre-construction survey to confirm whether Annex I habitats (cobble reef) are identified in the export cable corridor. Appropriate consideration shall be given to monitoring the presence of any invasive non-native species (INNS) and wider community type/structure, to identify changes that may affect site integrity. | Geophysical surveys as defined within the approved Geophysical (Bathymetrical) Monitoring Plan (SOWFL, 2019a) together with characterisation (DDC and grab) sampling are being undertaken in 2020 (See Section 4.2). |
|  |  |  |  To potentially microsite project infrastructure away from Annex I habitats, where required |  | Surveys undertaken will inform any mitigation plan in relation to habitats of conservation, ecological and/or economic importance (including Annex I habitats). | Areas of Focus have been identified (Sections 5 and 6) based on CEEI habitats that may potentially be affected by the construction and operation of SOWF (namely geogenic and biogenic reefs, hard substrate and the Dogger Bank SAC sandbank feature. The sampling and analysis methods proposed (Section 7) will also enable consideration of invasive species and community type/structure. |
|  |  |  |  Regional seabed monitoring to identify suitable areas within the array boundaries, for disposal of drill arisings/spoil should it be required |  |  |  |
|  |  | Post-construction | To validate the predictions made in the Environmental Statement and HRA with respect to potential effects on benthic Annex I habitats and linked receptor groups as relevant (Natural England and JNCC requirement) and to identify the presence of any non-native species and wider community/type structure | Dependent on the outcome of the surveys undertaken in condition 13(2)(a) above, appropriate surveys to determine the effects of construction activity on any benthic Annex I habitat in whole or in part inside the area(s) within the Order limits to validate predictions made in the Environmental Statement. |  | The objectives and pre construction benthic monitoring has been designed to consider the potential for post construction surveys. Data from both surveys would, if required, be used to validate the key predictions of the ES (Section 7) |
|  |  |  |  |  |  | The requirement for post construction monitoring would be confirmed during the reporting of the pre- construction benthic monitoring |
|  |  | Post-construction | Monitoring of any disposal mounds, generated through the deposition of drill arisings, to determine changes in size and form of mounds through the lifetime of the project, in order to inform any decommissioning requirements (Natural England and JNCC requirement) | Appropriate surveys to determine change in size and form of the disposal mounds over the lifetime of the authorised scheme. | Survey will only be required should drilling and subsequent in situ disposal be employed. The survey would constitute an adaptive management measure and would inform requirements for the mounds at the point of decommissioning, should this be required. Scope and timings to be agreed post consent but survey should consider a sample of disposal mounds where any mud or clay arisings are anticipated. This may require monitoring at years 1, 2, 3, 5, 10, 15 and 20 of the operational life of the project unless agreed otherwise with the MMO in consultation with the relevant SNCB. However, survey results will be reviewed with the MMO and relevant SNCBs and subsequently may not be required throughout the life of the project, depending on the persistence of any mounds produced. | The approach for monitoring of drill mounds has been discharged through the approval of the SOWFL Geophysical (Bathymetrical) Monitoring Plan (SOWFL, 2019a) |
|  |  |  |  | Note: this monitoring is repeated from Section 4.2 (marine physical processes). | Depending on the amount of drilling undertaken, it is expected that the number of locations monitored will be an agreed sample based/informed on/by the final project design and the environmental parameters, selecting locations across the range of different water depths and seabed habitat types experienced. |  |

EUNIS mapping

Figure B1 EUNIS classification mapping of the SOWF array area based on geophysical and benthic ecology data collected during the benthic characterisation surveys of the Dogger Bank Zone. Note that this is derived from the original MNCR mapping conducted during the EIA correlated to the EUNIS habitat classification.

Valued Ecological Receptor (VER) categories

Figure C1 Valued Ecological Receptor (VER) categories of the SOWF array area based on geophysical and benthic ecology data collected during the benthic characterisation surveys of the Dogger Bank Zone. Full details of VER categories can be found in Appendix E.

Valued Ecological Receptor (VER) protection status and conservation interest and their importance/value within the study area.

Table D1 Valued Ecological Receptor protection status, conservation interest and their importance/value within the study area.

Summary of predicted effects of SOWF on benthic habitats.

Table E1 Summary of predicted effects on SOWF on benthic habitats.

| Impacts | Mitigation | Residual impact |
| --- | --- | --- |
| Construction |  |  |
| Physical disturbance to habitats and species and temporary habitat loss | None | Negligible impact on all VER apart from VER C |
|  |  | Minor adverse impact on VER C |
| Increased suspended sediment concentration and sediment deposition | None | Negligible impact on all VER apart from VER C |
|  |  | Minor adverse impact on VER C |
| Release of sediment contaminants resulting in potential effects on benthic ecology | None | Negligible impact on VERs A, B and C in wind farm sites (and furthest offshore part of the cable corridor) |
|  |  | Minor adverse impact on VERs D to I (cable corridor VERs) |
| Increased suspended sediment concentration leading to impacts on plankton and primary productivity | None | Negligible impact |
| Physical disturbance to intertidal habitats and species during landfall works | None | Negligible impact (VERs H and I) |
| Potential construction phase impacts on Dogger Bank cSAC | None | Negligible impact (VER A and B) |  |
|  |  | Minor adverse impact (VER C) |  |
| Operation |  |  |  |
| Permanent loss of habitat via placement of project infrastructure (foundations, cable protection, scour protection) | None | Negligible impact on VERs D, E, F, G, H and I |  |
|  |  | Minor adverse impact on VERs A, B and C |  |
| Temporary impact on benthos due to physical disturbance caused by maintenance activities | None | Negligible impact |  |
| Change in hydrodynamics and inter-related effects on benthos | None | Negligible impact |  |
| Increase in suspended sediment concentration due to scour associated with foundations | None | Negligible impact for all VERs apart from VER C |  |
|  |  | Minor adverse impact for VER C |  |
| Increase in sediment deposition following increase in suspended sediment concentration due to scour associated with foundations | None | Negligible impact |  |
| Introduction of new habitat in the form of foundation structures, leading to potential colonisation. | None | Negligible impact |  |
| Effect of EMF on benthic communities | Where feasible cables will be buried to at least 1m | Negligible impact on all VERs |  |
| Potential operational phase impacts on Dogger Bank cSAC | None | Negligible impact (VER A and B) |  |
|  |  | Minor adverse impact (VER C) |  |
| Decommissioning |  |  |  |
| Increased suspended sediment concentration and sediment deposition | None | Negligible impact |  |
| Loss of species colonising hard structures | None | Negligible impact |  |
| Temporary disturbance to habitats via removal of cables | None | Negligible impact on all VER apart from VER C |  |
| Potential decommissioning phase impacts on the Dogger Bank cSAC | None | Negligible impact (VER A and B) |  |
|  |  | Minor adverse impact (VER C) |  |

Associated documents

Table G1 Documents relevant to the Benthic Monitoring Plan.

| Document | Document reference | Prepared by | Document type | Status |
| --- | --- | --- | --- | --- |
| Dogger Bank Teesside A and B Environmental Statement Chapter 12 Marine and Intertidal Ecology | 6.12 | Forewind | Consent application | Accepted (August 2015) |
| Dogger Bank Teesside A and B Offshore In Principle Monitoring Plan | F-EXL-DIX-010 | Forewind | Consent application | Accepted (August 2015) |
| Dogger Bank Teesside A and B Offshore Wind Farm – Non Material Change Application Regulation 63 of the Conservation of Habitats and Species Regulations 2017, and Regulation 28 of the Conservation of Offshore Marine Habitats and Species Regulations 2017 (The “Habitats Regulations”) |  | Department of Energy & Climate Change | Consent determination | Accepted (August 2015) |
| Record Of The Habitats Regulations Assessment Undertaken Under Regulation 61 Of The Conservation Of Habitats And Species Regulations 2010 (As Amended) And Regulation 25 Of The Offshore Habitats Regulations For An Application Under The Planning Act 2008 (As Amended) |  | Department for Business, Energy and Industrial Strategy | Consent determination | Accepted (August 2015) |
| SOWFL (2019a) Detailed Construction and Monitoring Programme: Geophysical (Bathymetrical) Monitoring Plan. | 003090255-04 | SOWFL | Approved Management Plan | Submitted to the MMO (Approved December 2019) |
| SOWFL (2019b) Sofia Offshore Wind Farm: Sofia Offshore Wind Farm: Project Update Meeting with Natural England: 20th August 2019: Benthic Note | 003235279-01 | SOWFL and Ocean Ecology Ltd. | Supporting document to inform discussions at workshop | Submitted to Natural England (August 2019) |
| SOWFL (2019c) Sofia Offshore Wind Farm: Benthic Monitoring Plan: Supporting Studies | 003097005-01 | SOWFL and Ocean Ecology Ltd. | Supporting document to inform scope of BMP | Submitted to the MMO and Natural England (November 2019) |
| Pre: construction plans and documentation: Detailed Construction and Monitoring Programme | 003454725-01 | SOWFL | Management plan for approval | This document will be submitted to the MMO for approval. |  |

final sampling design – nearshore (out to Kilometre Point 10)

The samples in the nearshore have been selected to gather baseline data on a representative number of features of each of the habitat(s) confirmed to be present within the proposed construction areas from the 2020 geophysical data and also include the two options for the HDD exit point that are currently being considered. The figures below show the proposed transects at the two potential HDD exit points and three further features of focus out to KP10. The table that follows provides the rationale for the choice of transect along with the coordinates for the start and end of each transect.

Figure I-1 Overview of nearshore BMP DDC transects overlain on 2020 SSS data.

Figure I-2 – Location of nearshore DDC transects relative to the proposed HDD exit point options overlain on 2020 MBES data.

Figure I-3– Location of nearshore DDC transects out to KP 10 overlain on 2020 MBES data.

Table I-2 – Rationale and coordinates of nearshore DDC transects (out to KP10).

| No. | Rationale | AOI | Start / End | Easting | Northing | KP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. Sandy sediment recorded during 2019 survey. | A | Start | 240359.58 | 6058249.48 | 1 |
| 1 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. Sandy sediment recorded during 2019 survey. | A | End | 240598.60 | 6058479.27 | 1 |
| 2 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. Sandy sediment recorded during 2019 survey. | A | Start | 240350.34 | 6058347.96 | 1 |
| 2 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. Sandy sediment recorded during 2019 survey. | A | End | 240561.67 | 6058169.46 | 1 |
| 3 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. No 2019 survey data coverage. | A/B | Start | 240416.00 | 6058412.59 | 1 |
| 3 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. No 2019 survey data coverage. | A/B | End | 240657.07 | 6058210.50 | 1 |
| 4 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. No 2019 survey data coverage. | B | Start | 240487.81 | 6058487.47 | 1 |
| 4 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. No 2019 survey data coverage. | B | End | 240757.61 | 6058263.84 | 1 |
| 5 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. Sandy sediment recorded during 2019 survey. | B | Start | 240483.50 | 6058167.15 | 1 |
| 5 | HDD area. Bedrock reef recorded during 2011/2012 EIA survey. Sandy sediment recorded during 2019 survey. | B | End | 240781.20 | 6058346.93 | 1 |
| 6 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef and clay exposure mosaic recorded during 2019 Site Charaterisation survey. | C | Start | 243971.13 | 6058380.37 | 4.8 |
| 6 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef and clay exposure mosaic recorded during 2019 Site Charaterisation survey. | C | End | 244259.62 | 6058380.20 | 4.8 |
| 7 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef and clay exposure mosaic recorded during 2019 Site Charaterisation survey. | C | Start | 244115.37 | 6058430.83 | 4.8 |
| 7 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef and clay exposure mosaic recorded during 2019 Site Charaterisation survey. | C | End | 244115.37 | 6058230.83 | 4.8 |
| 8 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | D | Start | 245257.73 | 6058379.62 | 6 |
| 8 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | D | End | 245559.11 | 6058379.45 | 6 |
| 9 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | D | Start | 245408.42 | 6058479.53 | 6 |
| 9 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | D | End | 245408.42 | 6058279.53 | 6 |
| 10 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | E | Start | 246863.17 | 6058377.60 | 7.5 |
| 10 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | E | End | 247063.17 | 6058377.48 | 7.5 |
| 11 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | E | Start | 246963.17 | 6058477.54 | 7.5 |
| 11 | Mixed sediment mapped during 2011/2012 EIA survey and bedrock / stony reef recorded during 2019 Site Charaterisation survey. | E | End | 246963.17 | 6058277.54 | 7.5 |

final sampling design – OFFSHORE (BEYOND Kilometre Point 10)

Feature of Focus I & III: Sediments and INNS

A combination of the updated habitat mapping (see Appendix L), existing EUSeaMap mapping available from EMODnet, the indicative scour assessment, and data from historical sampling stations were used to inform the selection of the sampling stations proposed to monitor potential impacts of FoF I & III. Once positioned, the stations were microsited to eliminate potential conflict with any UXO and archaeological targets identified during the 2020 site investigation survey programme. The proposed sampling stations accompanied by a rationale for the selection of each location are provided as Annex 1 (as a separate document).

WTGs

Sampling stations located at the WTG locations were selected based on the widest geographic spread of different broad scale habitats, combined with the varying levels of scour predicted across the SOWF array area. Predicted scour was based on preliminary shear stress analysis, presented in the form of heat maps, showing the percentage of time where the threshold for mobility for fine sand (d100 = 0.2 mm) was predicted to be exceeded during typical wintertime conditions. Fine sand and winter conditions were used as they represented the greatest range in % time of likely scour and therefore the ‘worst-case’ scour scenario. Each turbine location was assigned a scour score of high, medium, or low based on this analysis along with a dominant broad scale habitat allowing for a representative range of both scour and habitat type to be covered by the proposed sampling array.

Buffers were placed around the proposed WTG locations at 100 m, 200 m and 300 m. Eight stations were placed around the proposed WTG locations in a cruciform arrangement. The proposed cruciforms consist of ‘long arms’ aligned with the NW to SE tidal axis extending 300 m from each WTG location and ‘short arms’ aligned at right angles to the ‘long arms’ extending 100 m from each WTG location. Sampling stations were positioned at the 100 m, 200 m and 300 m points along each of the ‘long arms’ and at the 100 m point of the ‘short arms’.

ECR

ECR stations were positioned to align with historical sampling stations where possible. Stations were placed either side of the ECR at a distance of 15 m from the proposed cable. The range of broad-scale habitats and the geographical spread of historical sampling was also considered.

OCP

The OCP sampling consisted of one target which was fulfilled by selecting the central position of the proposed OCP. Buffers were placed around the proposed OCP at 100 m, 200 m and 300 m and same sampling arrangement was used as for the WTG locations (8 stations in cruciform arrangement).

Inter Array Cables (IAC)

IAC stations were positioned to align with historical sampling stations where possible. Stations were placed either side of the IACs at a distance of 15 m from the proposed cable. The range of broad-scale habitats and the geographical spread of historical sampling was also considered.

Control

To minimise potential conflicts and influence of other seabed infrastructure and activities (e.g. pipelines, subsea cables and other planned OWFs), nine broad areas were identified as free of conflicts throughout the Dogger SAC boundary but outside the SOWF and DBWF array and cable route boundaries and likely tidal excursion zones. The resulting polygons were used to position 12 suitable control stations. All historical sample data was filtered and the dominant broad-scale habitats identified within each polygon based on the updated habitat mapping. One station was selected within each of the 9 control areas to achieve a wide geographic spread of the dominant broadscale habitat types that are to be sampled within the SOWF area. One station was chosen based on the co-dominance of two broadscale habitat types. A further two stations were chosen to ensure the full range of available broad-scale habitats were covered. When possible, stations were selected at a central location within each polygon, away from the boundaries to further reduce any influence of other activities.

BRUVs

A subset of the proposed target stations located at the WTG, ECR and IAC locations were selected for BRUV deployments. The diversity of broadscale habitats, and the geographic spread was accounted for when selecting stations.  Both proposed ECR stations were placed on the same side of the proposed cable for continuity but the furthest away geographically. When selecting WTG stations the position of the OCP and IAC BRUV stations were considered to ensure a wide geographic spread across the array area.

Feature of Focus III: BIOGENiC/GEOGENIC REEF & INNS

All transects were positioned to target areas deemed to qualify as stony reef during the 2020 ground-truthing survey campaign that are also intersected by the proposed ECR route. Sites were selected for their geographical spread along the ECR as well as a combination of stony reef interpreted as either low or medium resemblance in line with Irving (2009). The proposed sampling stations accompanied by rationale are provided in Table J-3 whilst the GIS shapefiles are provided separately.

Senckenberg Research Institute (SRI) long-term and REWIND sampling design

Long-term sampling sites from SRI and Rewind, 37 and 45 stations respectively, were combined in order to conduct a micro-siting investigation across the proposed SOWF and surrounding area. This considered all proposed and current infrastructure including, Sofia and neighbouring Dogger Bank A, B and C wind farms and infrastructure, UXOs, subsea cables, oil and gas pipelines, and archaeological artifacts in order to mitigate for arising conflicts and ensure long-term sampling is conducted outside of any known nearshore impacts.

Stations were relocated if they occurred within 1km of most infrastructures, proposed and existing. Where this was not possible such as within the IAC, stations were relocated equidistant to infrastructures at the greatest distance possible. Care was taken to also not place relocated stations on top of other long-term/historical sampling sites. A total of 31 stations were relocated. The updated sampling stations accompanied by rationale are provided in Table K-4 whilst the GIS shapefiles are provided separately.

Table K-4 – Rationale and coordinates of SRI long-term monitoring and REWIND research cruises.

| Original Station No. | Updated Station No. | Rationale | Cruise | Orig_Lat | Orig_Long | New_Lat | New_Long |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 04 | 1 | Unchanged | REWIND | 54.927513 | 3.104996 | 54.927513 | 3.104996 |
| 05 | 2 | Unchanged | REWIND | 54.984243 | 2.946919 | 54.984243 | 2.946919 |
| 07 | 3 | Moved 210 m North to avoid conflict with Doggerbank C WTG | REWIND | 55.031268 | 2.810579 | 55.033160 | 2.810534 |
| 09 | 4 | Moved 942 m South to avoid conflict with Doggerbank C WTG | REWIND | 55.079369 | 2.676214 | 55.070906 | 2.676529 |
| 10 | 5 | Unchanged | REWIND | 55.144356 | 2.505294 | 55.144356 | 2.505294 |
| 01 | 6 | Unchanged | REWIND | 55.146050 | 3.190950 | 55.146050 | 3.190950 |
| 02 | 7 | Unchanged | REWIND | 55.075976 | 2.971619 | 55.075976 | 2.971619 |
| 12 | 8 | Unchanged | REWIND | 54.989911 | 2.675226 | 54.989911 | 2.675226 |
| 13 | 9 | Unchanged | REWIND | 54.939433 | 2.512210 | 54.939433 | 2.512210 |
| 11 | 10 | Moved to avoid conflict with Doggerbank C WTG | REWIND | 55.040893 | 2.576429 | 55.044190 | 2.566094 |
| 03 | 11 | Moved 540 m East to avoid conflict with Doggerbank C WTG | REWIND | 55.032967 | 3.063501 | 55.033070 | 3.071954 |
| 08 | 12 | Moved 1162 m NW to avoid conflict with Doggerbank C WTG | REWIND | 55.117809 | 2.829350 | 55.126301 | 2.818728 |
| 06 | 13 | Moved 770 m South to avoid conflict with Doggerbank C WTG | REWIND | 54.957022 | 2.805639 | 54.950098 | 2.805765 |
| 17 | 14 | Unchanged | REWIND | 55.172578 | 2.167407 | 55.172578 | 2.167407 |
| 18 | 15 | Moved 1179 m SW to avoid conflict with Sofia IAC and WTG | REWIND | 55.065228 | 2.196058 | 55.055203 | 2.190078 |
| 19 | 16 | Moved 898 m SW to avoid conflict with Sofia IAC and archaeological features | REWIND | 54.996146 | 2.223721 | 54.990655 | 2.213432 |
| 20 | 17 | Moved 1427 m South to avoid conflict with Sofia IAC and archaeological features | REWIND | 54.915589 | 2.253360 | 54.903096 | 2.248331 |
| 45 | 18 | Unchanged | REWIND | 54.794451 | 2.283988 | 54.794451 | 2.283988 |
| 23 | 19 | Moved 331 m SE to avoid conflict with Sofia IAC and archaeological features | REWIND | 55.001813 | 2.080465 | 54.999163 | 2.082845 |
| 15 | 20 | Unchanged | REWIND | 54.982542 | 2.377845 | 54.982542 | 2.377845 |
| 21 | 21 | Moved 1053 m SW to avoid conflict with Sofia IAC | REWIND | 54.925242 | 2.113068 | 54.919422 | 2.100097 |
| 16 | 22 | Moved 653 m NE to avoid conflict with Sofia IAC | REWIND | 55.057873 | 2.352158 | 55.061646 | 2.360001 |
| 14 | 23 | Moved 1398 m to avoid conflict with Dogger Bank C cable route | REWIND | 54.900254 | 2.382785 | 54.899960 | 2.404587 |
| 24 | 24 | Moved 1203 m NE to avoid conflict with Sofia IAC and WTG | REWIND | 55.069754 | 2.053789 | 55.080297 | 2.058005 |
| 32 | 25 | Unchanged | REWIND | 55.104812 | 1.453100 | 55.104812 | 1.453100 |
| 31 | 26 | Unchanged | REWIND | 55.020508 | 1.585489 | 55.020508 | 1.585489 |
| 30 | 27 | Unchanged | REWIND | 54.967232 | 1.671443 | 54.967232 | 1.671443 |
| 29 | 28 | Moved 215 m SE to avoid conflict with Dogger Bank B WTG | REWIND | 54.903662 | 1.777156 | 54.901863 | 1.778411 |
| 42 | 29 | Moved 3.8 km NE to avoid conflict with Dogger Bank C cable rote, Sofia cable route and charted cables | REWIND | 54.845680 | 1.892749 | 54.866592 | 1.940119 |
| 25 | 30 | Unchanged | REWIND | 55.130802 | 1.918437 | 55.130802 | 1.918437 |
| 27 | 31 | Unchanged | REWIND | 55.029003 | 1.740601 | 55.029003 | 1.740601 |
| 34 | 32 | Moved 189 m SE to avoid conflict with Dogger Bank B WTG | REWIND | 54.920132 | 1.559801 | 54.919019 | 1.562027 |
| 35 | 33 | Moved 1502 m South to avoid conflict with transmission cable | REWIND | 54.838284 | 1.412593 | 54.824784 | 1.413123 |
| 28 | 34 | Moved 579 m East to avoid conflict with Dogger Bank B WTG | REWIND | 54.976305 | 1.859158 | 54.976503 | 1.868197 |
| 33 | 35 | Moved 1216 m SE to avoid conflict with Dogger Bank B WTG and SEAL pipeline | REWIND | 54.971769 | 1.487679 | 54.969709 | 1.506335 |
| 26 | 36 | Moved 729 m North to avoid conflict with Dogger Bank B WTG | REWIND | 55.100290 | 1.684286 | 55.106835 | 1.683614 |
| 37 | 37 | Moved 978 m North to avoid conflict with Dogger Bank B WTG | REWIND | 54.866154 | 1.672431 | 54.874947 | 1.672142 |
| 36 | 38 | Unchanged | REWIND | 54.787045 | 1.571657 | 54.787045 | 1.571657 |
| 38 | 39 | Unchanged | REWIND | 54.789894 | 1.713926 | 54.789894 | 1.713926 |
| 41 | 40 | Moved 874 m SE to avoid conflict with Dogger Bank A WTG | REWIND | 54.764822 | 1.895713 | 54.757452 | 1.900421 |
| 43 | 41 | Moved 45 m NW to avoid conflict with Dogger Bank A WTG | REWIND | 54.791033 | 2.089356 | 54.791172 | 2.088688 |
| 40 | 42 | Unchanged | REWIND | 54.705499 | 1.949064 | 54.705499 | 1.949064 |
| 39 | 43 | Unchanged | REWIND | 54.674662 | 1.770240 | 54.674662 | 1.770240 |
| 44 | 44 | Unchanged | REWIND | 54.710065 | 2.133815 | 54.710065 | 2.133815 |
| 22 | 45 | Unchanged | REWIND | 54.916725 | 1.981667 | 54.916725 | 1.981667 |
| 3 | 46 | Unchanged | SRI | 54.604500 | 1.916667 | 54.604500 | 1.916667 |
| 4 | 47 | Unchanged | SRI | 54.479833 | 1.862000 | 54.479833 | 1.862000 |
| 5 | 48 | Unchanged | SRI | 54.618333 | 1.681667 | 54.618333 | 1.681667 |
| 6 | 49 | Unchanged | SRI | 54.757500 | 1.733333 | 54.757500 | 1.733333 |
| 7 | 50 | Moved 1044 m SE to avoid conflict with SEAL gas pipeline | SRI | 54.798167 | 1.453000 | 54.790352 | 1.462010 |
| 8 | 51 | Unchanged | SRI | 54.900000 | 1.316667 | 54.900000 | 1.316667 |
| 9 | 52 | Unchanged | SRI | 54.995667 | 1.652667 | 54.995667 | 1.652667 |
| 10 | 53 | Unchanged | SRI | 54.920833 | 1.779167 | 54.920833 | 1.779167 |
| 11 | 54 | Unchanged | SRI | 54.757833 | 2.007000 | 54.757833 | 2.007000 |
| 12 | 55 | Moved 279 m North to avoid conflict with a charted transmission cable | SRI | 54.691833 | 2.216167 | 54.694318 | 2.215546 |
| 13 | 56 | Move 2876 m NE to avoid conflict with charted gas pipelines | SRI | 54.433500 | 2.359667 | 54.444718 | 2.399628 |
| 14 | 57 | Unchanged | SRI | 54.515333 | 2.675833 | 54.515333 | 2.675833 |
| 15 | 58 | Unchanged | SRI | 54.663333 | 2.485000 | 54.663333 | 2.485000 |
| 16 | 59 | Unchanged | SRI | 54.790833 | 2.319167 | 54.790833 | 2.319167 |
| 17 | 60 | Moved 1540 m North to avoid conflict with Dogger Bank C export cable | SRI | 54.845667 | 2.086667 | 54.859507 | 2.086049 |
| 18 | 61 | Unchanged | SRI | 54.990167 | 2.091500 | 54.990167 | 2.091500 |
| 20 | 62 | Unchanged | SRI | 54.937500 | 2.328333 | 54.937500 | 2.328333 |
| 21 | 63 | Unchanged | SRI | 54.872500 | 2.604167 | 54.872500 | 2.604167 |
| 22 | 64 | Unchanged | SRI | 54.769000 | 2.542167 | 54.769000 | 2.542167 |
| 23 | 65 | Unchanged | SRI | 54.741000 | 2.767667 | 54.741000 | 2.767667 |
| 24 | 66 | Unchanged | SRI | 54.926667 | 2.950000 | 54.926667 | 2.950000 |
| 25 | 67 | Unchanged | SRI | 54.819000 | 3.055000 | 54.819000 | 3.055000 |
| 26 | 68 | Moved 316 m North to avoid conflict with cables | SRI | 54.845000 | 2.808333 | 54.847835 | 2.808052 |
| 27 | 69 | Unchanged | SRI | 54.950000 | 2.895000 | 54.950000 | 2.895000 |
| 28 | 70 | Unchanged | SRI | 55.004167 | 2.628333 | 55.004167 | 2.628333 |
| 29 | 71 | Unchanged | SRI | 55.127667 | 2.671667 | 55.127667 | 2.671667 |
| 30 | 72 | Unchanged | SRI | 55.208833 | 2.925000 | 55.208833 | 2.925000 |
| 31 | 73 | Unchanged | SRI | 55.093667 | 2.907333 | 55.093667 | 2.907333 |
| 32 | 74 | Unchanged | SRI | 54.924333 | 3.102333 | 54.924333 | 3.102333 |
| 33 | 75 | Unchanged | SRI | 55.024000 | 3.167000 | 55.024000 | 3.167000 |
| 34 | 76 | Unchanged | SRI | 55.108333 | 3.475833 | 55.108333 | 3.475833 |
| 35 | 77 | Unchanged | SRI | 55.176167 | 3.196000 | 55.176167 | 3.196000 |
| 36 | 78 | Unchanged | SRI | 55.306500 | 3.314167 | 55.306500 | 3.314167 |
| 37 | 79 | Unchanged | SRI | 55.385000 | 3.566667 | 55.385000 | 3.566667 |
| 38 | 80 | Unchanged | SRI | 55.376333 | 3.802500 | 55.376333 | 3.802500 |
| 39 | 81 | Unchanged | SRI | 55.482000 | 3.972333 | 55.482000 | 3.972333 |
| 40 | 82 | Unchanged | SRI | 55.459333 | 4.143667 | 55.459333 | 4.143667 |

Predictive HABITAT modelling report

- Provided under separate cover.

Registered office

Windmill Hill Business Park
Whitehill Way, Swindon, Wiltshire, SN5 6PB

Registered in England & Wales, Company Number 07791964

Based on SOWFL experiences in sampling the SOWF and wider Dogger Bank area in 2011/2012, 2020 and 2021, it is anticipated that samples volumes >5 L are unlikely due to the compact nature of the sandy sediments. As such, sample volumes > 3L will be deemed acceptable however the volume of subsamples taken for subsequent PSD analysis for samples between 3-5L will be kept to a minimum where possible.

Appendix C Ocean Ecology Limited. Sofia Offshore Windfarm Nearshore Export Cable Drop-Down Camera Survey Technical Report (Separate Document) (Ecodoc Ref: 003406742-01).

SOWFL will be submitting a dML variation request to include the Offshore Converter Platform within dML11 as a transmission asset due to its transfer to the OFTO

Where sufficient material is available in the vibrocore sample